Autonomous RC Car Using Two-Layer Edge Perception and Real-Time Control

Krishna Menon, Aditya Rana, and Parth Surve

Mukesh Patel School of Technology Management and Engineering, SVKM’s NMIMS University, Mumbai, India
[Email addresses / program details to be completed]

Abstract—This paper presents the design and software implementation of a small autonomous 4WD model car using a Raspberry Pi 5 and an Arduino Uno. The Pi is assigned camera-based perception and high-level decisions, while the Uno controls the motors and polls two forward-facing ultrasonic sensors. The repository contains a YOLO-based object-detection wrapper, a classical image-processing routine for pothole candidates, a three-zone hazard scorer, a KCF-based person tracker, Pi-side serial and navigation modules, Arduino motor-control sketches, and a documented command/telemetry protocol. The intended architecture combines visual context with local proximity sensing while separating perception workloads from motor actuation. This draft describes the design and implementation evidence currently present in the repository and defines an evaluation plan for the physical prototype. Repository code and design documentation do not, by themselves, verify end-to-end operation or performance. Accordingly, benchmark results, detection accuracy, response times, and safety claims are left for insertion only after repeatable tests on the assembled vehicle.

Index Terms—autonomous vehicle, edge computing, obstacle avoidance, Raspberry Pi, Arduino Uno, pothole detection, embedded vision.

# I. INTRODUCTION

Small autonomous vehicles must combine sensing, decision-making, and actuation under limits on cost, compute, power, and physical size. Camera-based vision can provide object categories and spatial context, whereas a range sensor can report nearby obstacles without semantic classification. The project described here uses a two-controller architecture: a Raspberry Pi 5 performs high-level perception and decision-making, and an Arduino Uno handles motor output and local ultrasonic sensing. The split is intended to keep motor control independent of the Pi’s vision workload; its practical benefit must be established through measurement.

## A. Problem Statement

The engineering problem is to integrate camera perception, close-range sensing, serial communication, and motor control into a 4WD skid-steer model car. Decisions generated by the Pi must be translated into bounded commands that the Uno can execute while maintaining a clear interface and predictable stop behavior. The system must be tested for functional correctness, communication behavior, obstacle response, and repeatability on the target hardware.

## B. Objectives

Define a modular two-controller architecture and the interfaces between perception, navigation, serial communication, and firmware.

Implement Pi-side visual detection, pothole candidate generation, zone scoring, and person tracking.

Implement Arduino-side motor actions, front ultrasonic sensing, and telemetry using a documented protocol.

Integrate the controllers and evaluate supported behavior using controlled bench and vehicle tests.

## C. Scope and Contributions

The repository targets a Raspberry Pi 5, an Arduino Uno based on the ATmega328P, an L298N driver, four TT motors arranged in left/right pairs, two front-facing HC-SR04P sensors, and a CSI camera. The codebase includes autonomous/reactive driving and follow-me paths. The intended contributions are a split-computing vehicle architecture, a Pi-to-Arduino serial interface, a perception pipeline combining learned detections and classical pothole candidates, and an evaluation framework. IMU-aided dead reckoning is not treated as complete: the sensor module describes ICM20948 heading support as a stub pending hardware confirmation.

# II. RELATED WORK

The final related-work section must synthesize peer-reviewed and authoritative sources on lightweight edge detection, embedded inference runtimes, road-damage perception, ultrasonic sensing, mobile robot avoidance, real-time microcontroller control, and heterogeneous Pi–MCU architectures. The capstone guidance calls for at least 15 references overall and detailed review of at least 10 journal or conference papers. This draft does not invent sources or bibliographic details; verified sources and numbered in-text citations must be added before submission. The expanded synthesis cites sources listed in IEEE order [1]–[20].

## A. Edge Vision for Mobile Robots

Compare lightweight object-detection models and runtimes using common dimensions: target processor, model size, input resolution, dataset, inference latency, throughput, memory use, and validation method. YOLOv10n and NCNN are described as the intended deployment path in repository documentation. Any assertion that this combination is faster or more accurate than alternatives must be supported by primary literature or a reproducible benchmark on the target Pi 5.

## B. Pothole and Road-Damage Detection

Review learned road-damage methods and classical image-processing techniques, focusing on dataset coverage, road texture, shadows, illumination, camera viewpoint, and the distinction between positive obstacles and negative obstacles. The repository’s Laplacian and morphology pipeline generates candidate pothole regions. Until it is evaluated against labeled examples, it should be characterized as a heuristic candidate detector rather than a validated classifier.

## C. Local Sensing and Real-Time Control

Prior work on ultrasonic sensing and microcontroller motor control can motivate a local reflex layer, but the trade-offs depend on sensor placement, timing, threshold selection, actuator characteristics, and tested environment. The project assigns ultrasonic polling and motor actuation to the Uno and semantic camera processing to the Pi. The research gap should be finalized only after the literature review compares prior systems and their validation evidence.

## D. Provisional Research Gap

A provisional gap statement is: there is a practical need to evaluate a low-cost educational vehicle that combines edge-based visual obstacle interpretation with independent microcontroller-based close-range sensing and timed motor control, while documenting end-to-end behavior on the assembled hardware. This statement must be refined and justified by the completed literature synthesis.

## II-E. Comparative Literature Synthesis and Research Gap

The development of single-stage object detectors provides the algorithmic basis for the project’s visual-perception component. YOLO formulates object detection as a unified image-to-box prediction problem [1], while YOLOv10 studies an end-to-end, non-maximum-suppression-free design and efficiency–accuracy trade-offs [2]. These papers support considering a compact detector for a mobile platform. They do not establish that a particular exported model meets latency or accuracy targets on this Raspberry Pi, camera, or road scene. Those are deployment questions that require on-device measurements.

Road-damage data and evaluation have progressed through shared datasets and challenges. RDD2022 describes a multi-national annotated dataset for road damage, including potholes [3], and CRDDC 2022 defines a challenge-based evaluation context [4]. KITTI demonstrates the value of a documented sensor and capture setup for mobile-robotics research [5]. These works inform the project’s data and validation choices, but dataset metrics cannot be transferred directly to a camera mounted close to the ground on a small RC chassis. Camera height, focal length, motion, road texture, and apparent defect scale alter the input distribution.

For tracking, KCF uses the structure of translated image patches and Fourier-domain operations to reduce the cost of maintaining a target location across frames [6]. The project pairs a KCF tracker with HSV appearance histograms for reacquisition. This is a practical lightweight design, but the tracker’s published speed does not predict identity accuracy in the capstone’s scene. The project must separately measure tracking continuity, loss, false reacquisition, and stop behavior. MobileNetV2 is another example of an architecture optimized around efficiency and accuracy trade-offs on resource-constrained devices [7]; it is a comparative design point, not an implemented detector in the current runtime.

Systems such as OpenBot and Duckietown demonstrate how accessible robot platforms can support autonomy education and research [8], [9]. Their relevance lies in modular interfaces, repeatable tasks, and transparent evaluation. The present vehicle uses a Raspberry Pi and an Arduino connected by a custom serial protocol rather than placing all sensing and control on one compute board or using a larger robotics middleware stack. ROS provides a widely used component and message architecture [10], while the project’s line-oriented serial protocol is intentionally narrower. The custom interface makes it necessary to test framing, acknowledgment handling, command replacement, timeouts, and recovery explicitly.

Local obstacle planning ranges from simple heuristics to methods that search dynamically feasible actions. The Dynamic Window Approach is a representative local collision-avoidance method [11]. The current implementation uses three horizontal image zones, a danger score, and a discrete CLEAR/LEFT/RIGHT/STOP result. It should therefore be described as a zone-scoring heuristic, not a trajectory planner or safety proof. In the pothole path, the classical detector uses Laplacian edges, thresholding, morphology, and contours; foundational work on edge detection and threshold selection provides context for these operations [12], [13], but does not validate this particular set of thresholds or its pothole recall.

The defensible research gap is an implementation-and-evaluation gap: the project integrates camera-based obstacle interpretation, front-facing ultrasonic telemetry, a serial command interface, and microcontroller-based motor control on an educational RC platform. The contribution should be evaluated through controlled end-to-end scenarios and a clear record of hardware, code revision, model, test conditions, measurements, and failures. The repository’s scenario sheets provide an initial test taxonomy, while the source code and documents provide traceability [20]. The training and architecture values in this report must be interpreted according to their stated evidence status.

# III. SYSTEM ARCHITECTURE

The vehicle consists of three functional blocks. The Pi captures camera frames, runs perception and tracking, computes a decision, and transmits motion commands. The Uno receives commands, drives the L298N channels, polls two forward-facing ultrasonic sensors, and sends acknowledgments and telemetry. Four motors are paired by vehicle side to enable skid-steer motion. The intended Pi–Uno connection is USB serial at 115200 baud, consistent with the repository’s accepted transport decision and protocol document.

## A. Hardware Configuration

The architecture documentation describes a Raspberry Pi 5 (8 GB), Arduino Uno, L298N, 4WD acrylic chassis, two HC-SR04P sensors, CSI camera, separate Pi and motor/Arduino power rails, and a common signal reference through the USB serial link. The exact camera module, board revisions, battery chemistry and capacity, converter, rail voltages, and physical grounding must be checked against the assembled vehicle. These details are not considered verified solely because they appear in design documentation. Verify motor-driver ratings against the L298 datasheet [15] and board specifications against the manufacturer documentation [16], [17].

## B. Pin Mapping

The architecture document lists motor enables on D5 and D6, direction inputs on D8–D11, and ultrasonic trigger/echo pairs on D7/D13 and D12/D4. Other repository text contains older conflicting pin assignments. Therefore, the final pin map must be reconciled against arduino/lib/pins.h, the active firmware sketch, and wire-by-wire inspection of the chassis before publication.

## C. Power and Electrical Design

Documentation describes separate supplies for the Pi and motor/Arduino subsystem to reduce coupling from motor transients, with a shared signal-ground reference through USB serial. The report should include a verified power diagram and actual component ratings. Do not describe the rails as electrically isolated unless the physical wiring supports that statement; a common ground is an intentional electrical connection.

# IV. PERCEPTION AND CONTROL METHODS

## A. Object Detection

The detector wrapper loads a configured YOLO model through Ultralytics and converts its output into records containing class, confidence, bounding box, and obstacle type. Repository assets include NCNN model files, but the active configuration, class names, image size, confidence threshold, export version, and successful inference on the physical Pi must be confirmed. Model availability in the repository is not evidence of a measured runtime. The intended NCNN inference runtime is documented by its upstream project [14].

## B. Classical Pothole Candidate Generation

The routine processes the lower portion of a BGR frame, converts it to grayscale, applies Gaussian smoothing, computes a Laplacian response, thresholds edges, applies morphological closing/opening, and filters contours by area and aspect ratio. It emits a pothole-labeled candidate with a fixed confidence value. This value is a heuristic constant, not a calibrated probability. Evaluation should report precision, recall, false positives, and missed detections on labeled road frames.

## C. Three-Zone Hazard Scoring

Each detection is assigned to a left, center, or right region according to its horizontal bounding-box center. Its danger score combines confidence, an obstacle-type weight, and a capped box-area fraction. The decision function selects CLEAR, LEFT, RIGHT, or STOP using configured thresholds. For the final experiment, document the exact code revision and parameters, and test obstacles in each region, including simultaneous blockage of both sides.

## D. Person Tracking

The tracking module uses OpenCV KCF and HSV appearance histograms for target re-identification after tracking loss. The design uses upper- and lower-body features with a configurable similarity threshold. Report the exact startup selection procedure and test target loss, occlusion, re-acquisition, similar clothing, and stop behavior. Do not claim reliable person following without repeated trials and defined metrics.

## E. Pi-Side Navigation and Arduino Firmware

The Pi control package includes a serial motor controller, PID steering, and navigation modules. The sensor interface reads ultrasonic values from Arduino telemetry. The IMU interface remains documented as a stub. Arduino firmware and the protocol describe timed DRIVE, REV, TURN, STOP, and latching stop/release messages, along with READY, ACK, state, ultrasonic, and completion telemetry. Confirm that active firmware and Pi parsing agree on command spelling, timing, state transitions, and baud rate.

# V. IMPLEMENTATION AND INTEGRATION

The intended runtime sequence is: initialize the camera, detector, serial controller, and sensor manager; wait for Arduino readiness; capture an image; detect objects and pothole candidates; score the zones; choose an action; send a bounded serial command; then parse telemetry and continue. Follow mode adds target tracking and visual steering. Timestamped frame and serial logs are needed to establish actual loop rate, end-to-end delay, and command handling behavior.

## A. Software Organization

The implementation is organized under car/perception for detection, avoidance, tracking, configuration, and sensor parsing; car/control for serial motor control, PID, and navigation; car/main.py for orchestration; and arduino for firmware, shared pin/protocol headers, and bench sketches. benchmarking/ contains inference benchmarking code and model assets. The report should identify the tested commit or source revision and record dependency/model versions.

## B. Validation Procedure

Validation should proceed from isolated bench checks to integration: verify firmware upload and motor directions; compare ultrasonic readings against measured distances; test each serial command and telemetry response; characterize bounded motor movements; then run constrained vehicle trials followed by controlled free-running trials. Record surface, lighting, obstacle dimensions, camera configuration, battery state, firmware and software revisions, and any safety constraint used during each test.

# VI. EXPERIMENTAL PLAN AND RESULTS

No numerical performance result is asserted in this draft. Repository scripts, scenarios, and architecture prose are supporting artifacts or proposed methods; they are not substitutes for dated logs from the physical Pi 5 and vehicle.

TABLE I. REPOSITORY-DOCUMENTED PROJECT PARAMETERS AND PERFORMANCE FIGURES

| Metric | Value and evidence status |
| --- | --- |
| Target compute and control | Raspberry Pi 5 (8 GB stated in architecture) + Arduino Uno / ATmega328P at 16 MHz [repository description; verify exact boards]. |
| Camera mode | 640 × 480 at 30 FPS appears in architecture documentation; current config sets camera FPS to 30 [verify active camera mode]. |
| Serial link | 115200 baud, 8-N-1; Pi timeout is 0.5 s [protocol/config]. USB serial is the accepted documented transport. |
| Motor PWM | 8-bit command range 0–255; D5/D6 Timer0 PWM stated as 980 Hz. Architecture values: forward 200, turn 210, reverse 190; main runtime uses cruise 180 and slow 120 [parameter mismatch; confirm active firmware and calibration]. |
| Ultrasonic response | Two front sensors. Thresholds differ across files: Arduino/protocol describes <20 cm, architecture/config state 25 cm, Pi navigation checks 25 cm [resolve before final]. Architecture states about 100 ms telemetry polling and 2-read debounce. |
| Avoidance timing | Firmware constants document reverse 400 ms, turn 500 ms, resume 2000 ms, and cooldown 500 ms. Turn calibration is 10 ms/degree and explicitly marked uncalibrated. |
| Pothole model training | results.csv contains 50 epochs. Epoch 1 to 50: box loss 2.01838 to 1.71172; class loss 6.06841 to 1.91510. Final recorded validation: precision 0.53992, recall 0.27092, mAP@0.5 0.25226, mAP@0.5:0.95 0.11421 [directly present in repository CSV; confirm dataset split/run metadata]. |
| Perception performance claims | Architecture/comparative report claims YOLOv10n NCNN 22–26 ms/frame (38–45 FPS), classical pothole CV <4 ms, and KCF >60 FPS. These are not independently supported by a dated on-device benchmark output in benchmarking/results; treat as unverified documentation claims. |
| Control/safety performance claims | Documents claim serial round trip about 1.2 ms, local reflex <10 ms, hardware PWM jitter <4 μs, and stopping distance <1.5 cm at 1.5 m/s. No corresponding measurement records were present in benchmarking/results; do not present these as validated results until traceable test evidence is added. |
| Motor/power estimates | Architecture estimates 2S pack 7.4 V nominal (8.4 V peak), about 2.0 V L298N drop, and about 5.4 V at the motor at nominal pack voltage. These are estimates; verify pack and measure actual loaded voltage. |
| Person re-identification | Current tracker code uses HSV histogram correlation threshold 0.55. Other architecture/report text describes different values (e.g., 0.72 or 0.80), so the value must be reconciled with the tested code revision. |
| Navigation estimate | navigation.py uses 0.35 m/s at PWM 180 and 25 cm obstacle abort; code comments explicitly label the speed calibration unverified. |

Note: Values marked as documentation claims or estimates are reproduced from project files, not confirmed by this report. The benchmark results directory currently has no run output. The training metrics above are transcribed from the repository CSV and should be checked against the exact training configuration.

## A. Test Matrix

1) Inference: record model/export, input size, warm-up, per-frame latency, throughput, CPU utilization, memory, temperature, and throttling state. 2) Pothole candidates: evaluate labeled frames and report precision, recall, and representative errors. 3) Ultrasonics: record raw readings at measured distances, threshold behavior, response time, and false triggers. 4) Serial control: record commands, ACK rate, parsing errors, response delay, disconnect behavior, and recovery. 5) Motor calibration: measure distance and turn angle versus PWM, duration, surface, and battery state. 6) Integrated avoidance: repeat clear path, left/right/center obstacle, both sides blocked, and obstacle during reverse scenarios. 7) Follow-me: measure tracking continuity, target-loss rate, re-acquisition outcome, and stop behavior.

## B. Results to Insert

[Insert measured data tables and plots with trial count, test conditions, measurement method, and software/firmware revision. Include raw benchmark output and serial logs in an appendix. Separate real-hardware results from offline video tests.]

## C. Analysis

Compare outcomes with the objectives and explain failure cases. Analyze disagreement between camera detections and ultrasonic readings, lighting and floor-surface effects, communication faults, and requested versus observed motion. State whether each result was repeatable and identify conditions outside the test scope. Cite applicable standards only when they were actually followed in the design or validation method.

# VII. ADVANTAGES, LIMITATIONS, AND APPLICATIONS

## A. Advantages

The architecture separates high-level perception from motor actuation responsibilities. Software modules isolate detection, tracking, zone scoring, serial communication, and navigation. The combination of camera-based detections and short-range ultrasonic data can provide complementary information when both subsystems are operational.

## B. Limitations

Two forward-facing sensors do not cover the rear and provide limited spatial detail. Classical edge and morphology rules can confuse cracks, shadows, and texture with potholes. Bounding-box size is only a rough distance proxy unless calibrated. KCF and color-based re-identification can fail under occlusion or appearance similarity. IMU integration, turn calibration, watchdog behavior, and pin-map consistency require confirmation. Performance also depends on battery condition, surface, lighting, camera settings, and software revision.

## C. Applications

After suitable validation, the prototype may support educational demonstrations of embedded vision, sensor integration, and hierarchical control, as well as controlled indoor obstacle-avoidance and person-following experiments. The current evidence does not establish road readiness or suitability for unsupervised operation.

# VIII. CONCLUSION AND FUTURE WORK

This paper describes a repository-backed design and implementation baseline for a two-layer autonomous RC car using a Raspberry Pi 5 and an Arduino Uno. The Pi-side code provides visual detection, pothole candidate generation, zone scoring, and tracking modules; the Arduino project defines motor control, ultrasonic sensing, and a serial interface. These artifacts establish an implementation direction but do not alone establish successful end-to-end operation or performance. Conclusions about objective completion must be updated using physical test evidence.

Future work includes confirming the wiring and power configuration; completing and validating IMU heading reads; calibrating motor speed, distance, and turn response; measuring pothole detector accuracy; testing communication-loss and stop behavior; resolving continuous steering versus timed motor-command behavior; and completing repeatable end-to-end trials on the assembled vehicle.

# EXPANDED ENGINEERING APPENDICES

The following appendices preserve detailed repository material that informs this report. They are included for traceability and team review. Architecture documents, diagrams, example telemetry, and scenario traces state intended or previously reported behavior; they are not independent evidence that the current vehicle has achieved those behaviors. When a source conflicts with current code or physical wiring, the conflict is retained and flagged for reconciliation rather than silently resolved.

# APPENDIX A — ARCHITECTURE DESIGN RECORD (REPOSITORY SOURCE)

Source-status note: this architecture document contains strong hardware, performance, and implementation claims, plus references to modules not present in the current file inventory. Treat it as a design record. Confirm every claim against current code, hardware, and run logs before using it as an achieved result.

## System Architecture — Autonomous Follow-Me and Obstacle-Avoidant RC Vehicle

Status: APPROVED / PRODUCTION BASELINE (Architecture v2 — Verified Hardware Baseline)

Last Updated: 2026-10-11

Target Hardware: Raspberry Pi 5 (8GB) + Arduino Uno R3 (ATmega328P)

Primary Repositories: model-car-capstone / MentorPi

Active Branch: pothole-detection / main

---

### Executive Summary

This document specifies the canonical system architecture for the autonomous, two-layer robotic model car developed for the B.Tech Capstone Project. The vehicle operates in two distinct operational regimes:

• Autonomous Navigation Mode: Cruising, dynamic obstacle avoidance, and camera-based negative obstacle (pothole/cavity) evasion.

• Follow-Me Mode: Target-locking, real-time person tracking via visual servoing, HSV appearance-based re-identification, and reflexive collision prevention.

To overcome the fundamental failure modes of monolithic, single-board robotics systems—namely Linux kernel preemption, software GPIO PWM jitter, and brownout vulnerability under heavy deep learning workloads—the architecture strictly partitions tasks into a Cognitive High-Level Brain (Raspberry Pi 5) and a Reflexive Hard Real-Time Spine (Arduino Uno ATmega328P).

+---------------------------------------------------------------------------------------+

SYSTEM TOPOLOGY

+---------------------------------------------------------------------------------------+

+----------------------------------+             +------------------------------+

|   Raspberry Pi 5 (High-Level)   |      |   Arduino Uno (Real-Time)   |

|   • CSI Camera (OV5647 5MP)   |   USB-Serial   |   • Hardware Timer0 PWM (D5/D6)   |

|   • YOLOv10n (NCNN NMS-Free)   |   115200 Baud   |   • 3-Tier Priority FSM   |

|   • RDD2022 Pothole YOLO / CV   |   8-N-1   |   • 2x HC-SR04P (D7/13, D12/4)   |

|   • HSV Appearance Re-ID Tracker   |   <----------->   |   • L298N Skid-Steer (Inverted)   |

|   • Discrete PID Steering Engine   |      |   • Auto-Brake (<25cm Reflex)   |

|      |      |   • Serial Halt ('s'/'g' Pause)   |

+----------------------------------+             +------------------------------+

^                                               ^

|   Isolated 5.1V/5A   |   7.4V (2S 18650)

+----------------------------------+             +---------------+--------------+

|   boAt EnergyShroom PB400 Battery   |      |   2S 18650 Li-Ion + Common GND   |

+----------------------------------+             +------------------------------+

+---------------------------------------------------------------------------------------+

---

### 1. Architectural Philosophy & The Two-Layer Split

### 1.1 The Single-Board Failure Mode (The Linux OS Problem)

Conventional edge AI mobile robots (e.g., NVIDIA JetBot, DonkeyCar) run both high-level neural inference and low-level motor pulsing on a single Linux system-on-chip. This design suffers from three critical failure modes:

• Software PWM Timing Jitter: The Linux Completely Fair Scheduler (CFS) provides no deterministic real-time execution guarantees. Under heavy neural inference (such as YOLOv10 or MobileNet), CPU interrupt latency surges from $<50\,\mu\text{s}$ to $>30\,\text{ms}$. Pulses generated across software GPIO pins jitter violently, causing motor shudder, differential wheel slip, and erratic skid-steer heading errors.

• Safety Reflex Preemption: When an obstacle appears within $15\,\text{cm}$, a single-board architecture must process the image frame, run bounding box prediction, score threat zones, and issue GPIO signals. This pipeline introduces $60\text{–}120\,\text{ms}$ of latency. At a cruising speed of $1.5\,\text{m/s}$, the chassis moves $18\,\text{cm}$ before braking initiates, guaranteeing a physical collision.

• OS Crash / Hang Vulnerability: In the event of a Linux kernel panic, out-of-memory (OOM) freeze, or Python process crash, GPIO pins latch in their last active logic state. A car driving forward will continue driving at full speed into walls, stairs, or human bystanders.

### 1.2 The Two-Layer Solution

Our architecture enforces strict physical and functional isolation:

Engineering Parameter   |   Single-Board Architecture (JetBot / DonkeyCar)   |   Our Two-Layer Architecture (Pi 5 + Arduino Uno)

Motor PWM Generation   |   Software GPIO (jittery under load, $\pm 25\,\text{ms}$)   |   Hardware 8-bit Timer0/Timer1/Timer2 (rock-solid, $< 4\,\mu\text{s}$)

Ultrasonic Echo Timing   |   User-space polling (unreliable, pulse truncation)   |   Dedicated microcontroller pulseIn() timing ($1\,\mu\text{s}$ precision)

Emergency Reflex Loop   |   Soft real-time (60–120 ms round trip)   |   Hard real-time Tier 2 reflex ($< 10\,\text{ms}$ autonomous trigger)

Crash Protection   |   Motors latch at previous state on crash   |   Hardware Watchdog halts all motors if Pi heartbeat lost ($> 500\,\text{ms}$)

Electrical Stability   |   Motor back-EMF couples directly into SBC power rail   |   Dual-rail isolated supplies eliminate brownout resets

BOM Cost   |   $\$250\text{–}\$350$ (expensive single boards / carrier kits)   |   $\sim \$120\text{–}\$150$ (accessible, modular academic components)

---

### 2. Hardware Architecture & Electrical Design

### 2.1 Complete Pin Mapping Matrix

> Hardware Status: ARCHITECTURE V2 VERIFIED & BENCH TESTED (2026-10-11).

> Complete physical pin tracing, ultrasonic transceiver mapping, motor polarity inversion correction, and TT motor PWM torque compensation have been verified on the physical chassis.

Authoritative pin definitions are maintained in [arduino/lib/pins.h](../arduino/lib/pins.h):

Arduino Uno Pin   |   Direction   |   Assigned Function   |   Connected Hardware Subsystem   |   Hardware Timer / Characteristics

D2   |   UNCONNECTED   |   Reserved / Unused   |   Physically Empty   |   Available for wheel encoder / INT0

D3   |   UNCONNECTED   |   Reserved / Unused   |   Physically Empty   |   Available for wheel encoder / INT1

D4   |   INPUT   |   Right Ultrasonic Echo   |   HC-SR04P Front-Right Sensor   |   $1\,\mu\text{s}$ pulse timing input (pulseIn)

D5   |   OUTPUT   |   ENA (Left Motor PWM)   |   L298N Dual H-Bridge Channel A   |   Timer0 (OC0B) — 8-bit PWM (0–255, 980 Hz)

D6   |   OUTPUT   |   ENB (Right Motor PWM)   |   L298N Dual H-Bridge Channel B   |   Timer0 (OC0A) — 8-bit PWM (0–255, 980 Hz)

D7   |   OUTPUT   |   Left Ultrasonic Trigger   |   HC-SR04P Front-Left Sensor   |   $10\,\mu\text{s}$ digital trigger pulse output

D8   |   OUTPUT   |   Motor IN1 (Left Dir 1)   |   L298N Dual H-Bridge Channel A   |   Direction bit 1 (Firmware inverted)

D9   |   OUTPUT   |   Motor IN2 (Left Dir 2)   |   L298N Dual H-Bridge Channel A   |   Direction bit 2 (Firmware inverted)

D10   |   OUTPUT   |   Motor IN3 (Right Dir 1)   |   L298N Dual H-Bridge Channel B   |   Direction bit 1 (Firmware inverted)

D11   |   OUTPUT   |   Motor IN4 (Right Dir 2)   |   L298N Dual H-Bridge Channel B   |   Direction bit 2 (Firmware inverted)

D12   |   OUTPUT   |   Right Ultrasonic Trigger   |   HC-SR04P Front-Right Sensor   |   $10\,\mu\text{s}$ digital trigger pulse output

D13   |   INPUT   |   Left Ultrasonic Echo   |   HC-SR04P Front-Left Sensor   |   $1\,\mu\text{s}$ pulse timing (Uno 'L' LED flicker is purely cosmetic)

#### 2.1.1 The Ultrasonic Pin Resolution & Phantom IR Removal

Initial documentation and legacy sketch comments incorrectly assumed pins D2 and D3 were connected to digital IR obstacle sensors, while reversing ultrasonic sensor channel mappings. Physical signal tracing and diagnostic bench testing confirmed:

• Pins D2 and D3 are physically empty on the chassis. No IR sensors are installed on the chassis; D2 and D3 are designated as unreserved inputs, preserving them for future quadrature wheel encoders or external hardware interrupts.

• The real physical pin routing is verified as:

* Left Ultrasonic Sensor: Trigger = D7, Echo = D13

* Right Ultrasonic Sensor: Trigger = D12, Echo = D4

• Pin D13 Characteristics: On the Arduino Uno, pin D13 is internally tied via an op-amp/resistor circuit to the onboard 'L' surface-mount LED. When the Left Ultrasonic emits an echo pulse, the LED flickers faintly. This is cosmetic only and introduces zero signal distortion or timing inaccuracy to pulseIn() timing.

• Live simultaneous centimeter distance telemetry streaming was bench-validated from both ultrasonic transceivers at 115200 baud.

#### 2.1.2 Motor Wiring Polarity & Direction Inversion

Due to physical chassis mounting orientations, the 4WD TT gearbox DC motors were wired with inverted polarity relative to standard H-bridge logic (asserting IN1=HIGH, IN2=LOW drove the chassis in reverse). Under the project's strict hard constraint (borrowed lab hardware under shared ID rotation; no cutting or desoldering), physical rewiring was rejected.

This was resolved cleanly at the firmware layer via #define INVERT_MOTOR_POLARITY true in [arduino/lib/pins.h](../arduino/lib/pins.h) and const bool INVERT_DIR = true; in sketch files. The motor driver routines invert speed sign (s = -s) prior to setting IN1–IN4, ensuring all high-level forward drive commands spin the wheels forward.

#### 2.1.3 TT Motor Torque & L298N Voltage Drop Calibration

The L298N dual H-bridge motor driver employs internal bipolar Darlington transistors ($V_{CE(\text{sat})} \approx 1.8\text{–}2.0\,\text{V}$ total drop across high-side and low-side transistors). When powered by a 2S 18650 Li-Ion battery pack ($7.4\,\text{V}$ nominal, $8.4\,\text{V}$ peak), the effective motor terminal voltage is only:

$$V_{\text{motor}} = V_{\text{battery}} - V_{\text{drop}} \approx 7.4\,\text{V} - 2.0\,\text{V} = 5.4\,\text{V}$$

The legacy default PWM speed value of $120$ ($\approx 47\%$ duty cycle) delivered an effective RMS voltage of $\sim 2.5\,\text{V}$, resulting in severe motor stall under the chassis curb weight and floor friction. Torque has been systematically calibrated and empirically verified on the chassis:

• Forward Cruising (SPEED_FORWARD): PWM 200 ($\sim 78\%$ duty cycle, $\sim 4.2\,\text{V}$ RMS)

• Pivot Skid-Turn (SPEED_TURN): PWM 210 ($\sim 82\%$ duty cycle, delivers robust breakaway torque for zero-radius turns)

• Reverse Clearance (SPEED_REVERSE): PWM 190 ($\sim 75\%$ duty cycle)

### 2.2 Dual-Rail Isolated Power Distribution

To prevent inductive motor back-EMF and switching transients from triggering Raspberry Pi brownouts, the power system is partitioned into two physically isolated rails:

MOTOR & ARDUINO RAIL (Common Ground)               PI 5 RAIL (Isolated)

====================================               ====================

+-------------------------------+                 +-----------------------------------+

| 2S 18650 Li-Ion Battery Pack  |                 | boAt EnergyShroom PB400 Powerbank |

|   7.4V Nominal (1500-2500mAh) |                 |   USB-C 5.1V / 5.0A Output        |

+---------------+---------------+                 +-----------------+-----------------+

|                                                   |

|                                                   v

|                                 +-----------------+-----------------+

|                                 | Raspberry Pi 5 (8GB)              |

|                                 | Broadcom BCM2712 Quad-Core A76    |

|                                 +-----------------+-----------------+

|                                                   |

v                                                   | USB-A to USB-B

+------+------+                                            | Serial Cable

|    L298N    |                                            | (Data + Common GND)

|    Motor    |<-------------------------------------------+

|    Driver   |

+---+----+----+

|    |

7.4V VMS |    | Logic 5V / Common GND

v    v

+-----+----+----+

|  Arduino Uno  |

|  ATmega328P   |

+-------+-------+

|

v

+-------+-------+

| 16-Pin Berg   |

| Strip Header  |

+-------+-------+

|

+-------+-------+

|               |

v               v

4x TT DC        2x HC-SR04P

Motors          Ultrasonics

(Skid-Steer)      (Front Array)

• Common Ground Principle: The motor power rail (2S 18650 negative terminal, L298N GND, and Arduino GND) shares a single ground reference with the Raspberry Pi 5 exclusively through the shielded USB-Serial cable. This preserves 5V logic signal integrity while completely isolating high-current inductive motor spikes from the Pi's sensitive power rail.

• 16-Pin Berg Strip Header: Provides organized distribution of the regulated 5V bus and ground reference:

* Pins 1–8: Common +5V Rail (Arduino 5V output, 2x HC-SR04P VCC, L298N logic VCC, spare headers).

* Pins 9–16: Common GND Rail (Arduino GND, 2x HC-SR04P GND, L298N logic GND, spare headers).

### 2.3 Sensor Array & Spatial Field-of-View (FOV)

The chassis mounts an overlapping multi-sensor array:

FRONT OF CHASSIS

______________________________________

|                                      |

|   [US1: Left]          [US2: Right]  |

|   (Trig D7,            (Trig D12,    |

|    Echo D13)            Echo D4)     |

|      \                    /          |   US1: 15° Left Divergence

|       \   Forward Cone   /           |   US2: 15° Right Divergence

|        \   Overlapping  /            |   Combined Cone: ~60°

|         \              /             |   Threshold: < 25.0 cm (Auto-Brake)

|          \            /              |   Debounce: 2 Confirm Reads

|           \          /               |

|            \        /                |

|             \      /                 |

|               [CAM]                  |   CAM: CSI Camera (OV5647, ~54° HFOV)

|               (CSI)                  |   (D2/D3 empty; no IR sensors)

|                                      |

|  [FL Wheel]          [FR Wheel]      |   Skid-Steer TT DC Motors

|  [RL Wheel]          [RR Wheel]      |   Left Pair: ENA(D5) / IN1(D8) / IN2(D9)

|______________________________________|   Right Pair: ENB(D6) / IN3(D10) / IN4(D11)

REAR OF CHASSIS

• HC-SR04P Ultrasonic Transceivers (x2): Splayed laterally by $15^\circ$ to cover corner obstacles. Effective ranging: $2\,\text{cm}$ to $300\,\text{cm}$. The reflex auto-brake threshold is calibrated at $25.0\,\text{cm}$ (OBSTACLE_CM = 25) with a 2-sample consecutive debounce filter (CONFIRM_READS = 2) across $60\,\text{ms}$ loop cycles to suppress acoustic multipath spikes.

• CSI Monocular Camera (OmniVision OV5647 5MP Rev 1.3): Replaces the damaged Global Shutter camera, interfacing via a 22-pin to 15-pin FPC adapter cable. Features a $53.5^\circ$ horizontal FOV ($3.6\,\text{mm}$ focal length, $1/4''$ sensor). Mounted $12\,\text{cm}$ above the ground plane, angled downwards at $15^\circ$ pitch to cover both horizon targets and road surface ($0.3\text{–}4.0\,\text{m}$). Diagnostic testing is supported via the local low-latency MJPEG streaming utility ([tools/stream.py](../tools/stream.py)).

• Sensor Blind Spot Coverage: Horizontal ultrasonic sensors cannot detect road depressions or potholes. Conversely, monocular vision struggles with transparent glass or untextured surfaces at $<10\,\text{cm}$. The multi-modal sensor fusion completely eliminates mutual blind spots.

---

### 3. Firmware Architecture & 3-Tier Reflex State Machine

The Arduino Uno firmware executes a deterministic, non-blocking finite state machine at a calibrated loop rate. The verified diagnostic sketch ([arduino/motor_test/dual_motor_speed_test.ino](../arduino/motor_test/dual_motor_speed_test.ino)) and production firmware ([arduino/point_to_point_control/point_to_point_control.ino](../arduino/point_to_point_control/point_to_point_control.ino)) implement the 3-tier priority hierarchy:

stateDiagram-v2

[*] --> STATE_IDLE : Power On / [TELEM] READY

STATE_IDLE --> STATE_DRIVING : DRIVE cmd / 'g' Resume

STATE_IDLE --> STATE_TURNING : TURN cmd

STATE_IDLE --> STATE_REVERSING : REV cmd

STATE_DRIVING --> STATE_IDLE : Timer Expires / [TELEM] DONE DRIVE

STATE_TURNING --> STATE_IDLE : Timer Expires / [TELEM] DONE TURN

STATE_REVERSING --> STATE_IDLE : Timer Expires / [TELEM] DONE REV

STATE_DRIVING --> STATE_AVOIDING : US reading < 25cm (Tier 2 Reflex)

STATE_TURNING --> STATE_AVOIDING : US reading < 25cm (Tier 2 Reflex)

STATE_REVERSING --> STATE_AVOIDING : US reading < 25cm (Tier 2 Reflex)

STATE_AVOIDING --> STATE_IDLE : Avoidance Sequence Done / [TELEM] DONE AVOID

STATE_DRIVING --> STATE_PI_STOPPED : PI_STOP / Serial 's' (Tier 1 Latch)

STATE_TURNING --> STATE_PI_STOPPED : PI_STOP / Serial 's' (Tier 1 Latch)

STATE_REVERSING --> STATE_PI_STOPPED : PI_STOP / Serial 's' (Tier 1 Latch)

STATE_AVOIDING --> STATE_PI_STOPPED : PI_STOP / Serial 's' (Tier 1 Latch)

STATE_IDLE --> STATE_PI_STOPPED : PI_STOP / Serial 's' (Tier 1 Latch)

STATE_PI_STOPPED --> STATE_IDLE : PI_GO / Serial 'g' (Latch Released)

### 3.1 Priority Tiers

#### Tier 1: Emergency Latches, Serial Pause & Watchdog (Absolute Highest Priority)

• PI_STOP / Serial Emergency Halt ('s'): Instantaneous latch. Forces ENA=0, ENB=0 (stopAll()), and transitions to STATE_PI_STOPPED (or isStopped = true). All ongoing actions (including mid-maneuver Tier 2 avoidance) are aborted immediately. The vehicle remains locked until a valid release command is received:

* Over USB-Serial protocol: PI_GO command releases the latch to STATE_IDLE.

* Over interactive serial monitor: Sending 'g' / 'G' resumes operation.

• Watchdog Failsafe: The firmware monitors incoming serial timestamps. If no valid packet is received within $5000\,\text{ms}$ (WATCHDOG_TIMEOUT_MS), the car halts all motors and emits [TELEM] HEARTBEAT_TIMEOUT.

• Command Blocking: Any Tier 3 movement command received during STATE_PI_STOPPED is rejected with [ACK] BLOCKED (PI_STOPPED).

#### Tier 2: Hard Real-Time Local Avoidance Reflex (Autonomous Microcontroller Loop)

Triggered autonomously whenever either front ultrasonic sensor reads $< 25.0\,\text{cm}$ (OBSTACLE_CM = 25) confirmed over 2 consecutive readings (CONFIRM_READS = 2). Operates with zero latency independent of the Raspberry Pi:

• Case A: Both Sensors Blocked ($dL < 25\,\text{cm}$ and $dR < 25\,\text{cm}$):

* Phase 1 — Reverse (REVERSE_MS = 400ms): Reverses both motors at PWM 190 (SPEED_REVERSE) to clear the collision zone.

* Phase 2 — Pivot Turn (BOTH_TURN_MS = 450ms): Evaluates sensor clearance and turns toward the more open side ($dL \ge dR \rightarrow$ Turn Left; $dL < dR \rightarrow$ Turn Right) at PWM 210 (SPEED_TURN).

• Case B: Right Sensor Blocked Only ($dR < 25\,\text{cm}$):

* Immediately executes a fast skid-turn Left (setLeft(-210), setRight(210)).

• Case C: Left Sensor Blocked Only ($dL < 25\,\text{cm}$):

* Immediately executes a fast skid-turn Right (setLeft(210), setRight(-210)).

• Case D: Path Clear ($dL \ge 25\,\text{cm}$ and $dR \ge 25\,\text{cm}$):

* Drives forward at calibrated speed PWM 200 (SPEED_FORWARD).

• Phase 3 — Forward Resume & Cooldown: Resumes forward cruising. Cooldown period ($500\,\text{ms}$) suppresses re-triggering during initial acceleration.

• PI_AVOID Hint Override: If the Pi previously provided an avoidance hint (PI_AVOID LEFT or PI_AVOID RIGHT), that direction strictly overrides the local decision for both-blocked conditions.

#### Tier 3: High-Level Navigation (Supervised by Raspberry Pi)

Executes timed movement primitives calibrated for the 4WD skid-steer chassis:

• DRIVE <speed> <duration_ms> (Default cruising speed: PWM 200)

• REV <speed> <duration_ms> (Default reverse speed: PWM 190)

• TURN <angle_deg> <speed> (Negative angle = Left, Positive angle = Right; speed: PWM 210)

• STOP (Non-latched immediate halt to STATE_IDLE)

---

### 4. Perception & Computer Vision Architecture

The perception pipeline running on the Raspberry Pi 5 combines deep neural detection, a classical computer vision fallback for road cavities, and an appearance-based person tracker.

graph TD

CAM["CSI Camera Ingest<br/>(640x480 @ 30 FPS)"] --> PRE["Preprocessing & ROI Extraction<br/>(Bottom 55% Road Plane)"]

PRE --> YOLO["YOLOv10n (Tencent NCNN)<br/>Positive Obstacles (Cars, People)"]

PRE --> POTHOLE["RDD2022 Custom YOLO / Classical CV<br/>Negative Obstacles (Potholes, Cracks)"]

YOLO --> MERGE["Detection Fusion & Zone Spatial Corridors"]

POTHOLE --> MERGE

MERGE --> ZONE["Zone Hazard Scorer<br/>(Left, Center, Right)"]

ZONE --> DEC["Decision Arbiter Matrix"]

DEC -->|"EMERGENCY DANGER"| PISTOP["PI_STOP (Latch Car)"]

DEC -->|"DIRECTIONAL SWERVE"| SWERVE["PI_AVOID + Timed TURN"]

DEC -->|"CLEAR PATH"| FOLLOW["Follow-Me Visual Servoing"]

FOLLOW --> KCF["KCF Spatial Tracker"]

FOLLOW --> HSV["HSV Torso Appearance Re-ID"]

KCF --> PID["Discrete PID Controller"]

PID --> NAV["Serial Command Stream<br/>(DRIVE / TURN to Arduino)"]

### 4.1 Positive Obstacle Detection: YOLOv10n under Tencent NCNN

• Architecture: YOLOv10n eliminates the Non-Maximum Suppression (NMS) post-processing bottleneck by incorporating dual-label assignments during training.

• Inference Engine: Tencent NCNN framework with ARM Cortex-A76 NEON FP16 vectorization.

• Target Classes: Person (Class 0), Bicycle (1), Car (2), Motorcycle (3), Bus (5), Truck (7).

• Latency Profile: $\sim 22\text{–}26\,\text{ms}$ on physical Pi 5 hardware (enabling sustained 30+ FPS pipelines).

### 4.2 Negative Obstacle & Pothole Detection: Dual-Path Engine

Negative obstacles are depressions in the road plane that cannot reflect horizontal acoustic waves. Our system provides two complementary detection layers:

#### Path A: Custom RDD2022 YOLOv8n Deep Model (pothole_detection/)

• Training Dataset: Road Damage Dataset 2022 (RDD2022 India Sub-dataset, Class D40 Pothole).

• Weights: [pothole_detection/best.pt](../pothole_detection/best.pt) and [pothole_detection/rdd2022_yolov8n.pt](../pothole_detection/rdd2022_yolov8n.pt).

• Dynamic Corridor & Directional Swerve Module: Implemented in [pothole_detection/run_detection.py](../pothole_detection/run_detection.py):

* Trapezoidal Perspective ROI isolates the active driving corridor.

* Zone partitioning: Left Zone ($< 33\%$), Center Danger Corridor ($33\%\text{–}67\%$), Right Zone ($> 67\%$).

* Automated swerve trajectory selection (SWERVE_LEFT, SWERVE_RIGHT, EMERGENCY_STOP).

#### Path B: Classical Computer Vision Fallback (cv_detect_potholes)

Implemented in [car/perception/detector.py](../car/perception/detector.py) and [pothole_detection/negative_obstacle_detection/detector.py](../pothole_detection/negative_obstacle_detection/detector.py). Runs in $<4\,\text{ms}$ with zero GPU/NPU dependency:

• ROI Cropping: Extracts bottom 55% of the frame (roi_y = int(h * 0.45)).

• Gaussian Pre-Filtering: $5\times 5$ Gaussian kernel suppresses high-frequency asphalt noise.

• Laplacian Edge Extraction: Computes second-order spatial derivatives (cv2.Laplacian(blur, cv2.CV_64F)). Sharp intensity changes at cavity boundaries yield high response magnitudes.

• Morphological Elliptical Structuring: Applies morphological closing followed by opening with a $13\times 13$ elliptical structuring element (cv2.MORPH_ELLIPSE) to solidify cavity regions while rejecting linear surface cracks.

• Contour Geometry Screening:

* Reject if $\text{Area} < 900\,\text{px}$ (small surface debris).

* Reject if $\text{Area} > 0.25 \times \text{ROI Area}$ (broad shadows or lighting gradients).

* Reject if aspect ratio $\text{Width}/\text{Height} \notin [0.25, 5.0]$.

### 4.3 Hazard Zone Danger Scoring

In [car/perception/avoidance.py](../car/perception/avoidance.py), bounding boxes are transformed into localized danger scores:

$$\text{Danger} = \text{Confidence} \times W \times \left(0.4 + \min\left(\frac{\text{Area}_{\text{box}}}{\text{Area}_{\text{frame}}}, 0.5\right)\right)$$

• $W = 1.0$ for standard positive obstacles.

• $W = 1.2\text{ to }2.0$ (POTHOLE_WEIGHT) for negative obstacles, reflecting the severe hazard of chassis drop-off and high-centering.

### 4.4 Follow-Me Person Tracking & Appearance Re-Identification

Implemented in [car/perception/tracker.py](../car/perception/tracker.py):

• Tracking Subsystem: OpenCV Kernelized Correlation Filter (KCF) for high-speed frame-to-frame bounding box tracking ($> 60\,\text{FPS}$).

• HSV Torso Appearance Model: On initial target lock, a normalized 2D HSV color histogram ($H \times S$, $30 \times 32$ bins) is computed from the upper-torso region of the target bounding box.

• Re-Acquisition Engine: If KCF tracking confidence drops below threshold (e.g., target occluded behind a pillar or walked out of frame during sharp pivot):

1. Car halts motion (PI_STOP) to stabilize the camera feed.

2. YOLOv10n scans all candidates in frame.

3. The re-ID module evaluates candidate histograms using Bhattacharyya distance:

$$d(H_1, H_2) = \sqrt{1 - \sum_i \frac{\sqrt{H_1(i) \cdot H_2(i)}}{\sqrt{\sum H_1 \cdot \sum H_2}}}$$

4. The candidate with the highest similarity exceeding the re-ID threshold is re-locked as the target owner.

• Discrete PID Visual Servoing: [car/control/pid.py](../car/control/pid.py) modulates steering angle and motor PWM to maintain a $1.2\,\text{m}$ standoff distance:

* Steering error: Horizontal offset of bounding box center from frame centerline.

* Distance error: Deviation of bounding box height from calibrated $30\%$ frame height baseline.

---

### 5. Serial Telemetry & Command Protocol

Authoritative Specification: [arduino/PROTOCOL.md](../arduino/PROTOCOL.md)

• Transport: USB-Serial CDC (/dev/ttyUSB0 or /dev/ttyACM0)

• Baud Rate: 115200 bps, 8 data bits, no parity, 1 stop bit (8-N-1)

• Framing: Plaintext ASCII terminated with newline (\n)

• Round-Trip Latency: $\sim 1.2\,\text{ms}$ transaction time

### 5.1 Command Set (Raspberry Pi $\rightarrow$ Arduino)

Command   |   Syntax   |   Parameters   |   Tier   |   Functional Description

PI_STOP   |   PI_STOP   |   None   |   1   |   Latches vehicle in STATE_PI_STOPPED. Forces $PWM=0$. Overrides all states.

PI_GO   |   PI_GO   |   None   |   1   |   Releases PI_STOP latch. Returns car to STATE_IDLE.

PI_AVOID   |   PI_AVOID <dir>   |   LEFT or RIGHT   |   1   |   Stores one-shot direction hint for the next Tier 2 reflex avoidance.

's' / 'S'   |   s   |   Single char   |   1   |   Interactive emergency halt: immediately zeroes all motor outputs.

'g' / 'G'   |   g   |   Single char   |   1   |   Interactive resume: unpauses car and resumes sensor-driven execution.

DRIVE   |   DRIVE <spd> <ms>   |   spd: 0–255, ms: Duration   |   3   |   Drives forward at PWM speed for specified duration, then halts to IDLE.

REV   |   REV <spd> <ms>   |   spd: 0–255, ms: Duration   |   3   |   Reverses at PWM speed for specified duration, then halts to IDLE.

TURN   |   TURN <deg> <spd>   |   deg: -180 to +180, spd: PWM   |   3   |   Skid-steer turn by angle ($<0$ Left, $>0$ Right) at specified PWM speed.

STOP   |   STOP   |   None   |   3   |   Immediate non-latched halt to STATE_IDLE.

PING   |   PING   |   None   |   3   |   Liveness health check. Returns [ACK] PONG.

### 5.2 Telemetry Packet Set (Arduino $\rightarrow$ Raspberry Pi)

Message Header   |   Format Example   |   Frequency   |   Functional Description

[TELEM] READY   |   [TELEM] READY Uno-ATmega328P   |   Boot   |   Emitted upon microcontroller reset and setup completion.

[TELEM] STATE   |   [TELEM] STATE=DRIVING SPEED=200   |   On change   |   Signals FSM state transition and active drive speed.

[TELEM] US   |   [TELEM] US LEFT=24.5 RIGHT=88.2   |   Every $100\,\text{ms}$   |   Calibrated distance readings from front ultrasonic sensors.

Diagnostic Stream   |   L: 24 cm \   |   R: 88 cm --> CLEAR: Driving FORWARD   |   Loop ($60\,\text{ms}$)   |   Human-readable telemetry emitted by diagnostic obstacle avoidance firmware.

[TELEM] AVOID   |   [TELEM] AVOID RIGHT   |   Event   |   Emitted when Tier 2 reflex initiates an evasive maneuver.

[TELEM] DONE   |   [TELEM] DONE DRIVE   |   Event   |   Confirms completion of a timed movement primitive.

[ACK]   |   [ACK] DRIVE 200 2000   |   Immediate   |   Confirms syntax validity and acceptance of command.

[ACK] BLOCKED   |   [ACK] BLOCKED (PI_STOPPED)   |   Immediate   |   Emitted when command rejected due to active Tier 1 latch.

---

### 6. System Scenarios & Behavioral Sequence

The system architecture handles complex real-world conditions through 10 formal behavioral scenarios documented in [docs/scenarios/](../docs/scenarios/):

### Scenario 05: Negative Obstacle / Pothole Ahead (Camera Detection & Bypass)

• Situation: Vehicle is cruising forward. Both ultrasonic sensors read clear ($>90\,\text{cm}$ because horizontal sound waves fly across ground depressions). The Pi camera detects a pothole $1.5\,\text{m}$ ahead.

• Message Sequence:

1. Pi vision detects road cavity via custom RDD2022 YOLO or classical CV.

2. Pi sends PI_STOP $\rightarrow$ Arduino immediately halts motors ($PWM=0$) and acknowledges [ACK] PI_STOP.

3. Pi analyzes Left vs. Right clearance in frame, determining Left is clear.

4. Pi sends PI_AVOID LEFT hint (stored in Arduino memory).

5. Pi sends PI_GO $\rightarrow$ Arduino releases emergency latch to STATE_IDLE.

6. Pi executes bypass sequence: TURN -60 150 (steer left), waits for [TELEM] DONE TURN, sends DRIVE 150 2000 (clear hole), sends TURN 60 150 (straighten path), and resumes cruising with DRIVE 180 5000.

### Scenario 06: PI_STOP Preemption During Tier 2 Reflex Avoidance

• Situation: The car is in the middle of an autonomous Tier 2 ultrasonic evasive maneuver (skid-turning away from a wall) when the Pi camera detects an approaching human or dynamic vehicle in the new heading.

• Resolution: PI_STOP has absolute priority over Tier 2. The moment PI_STOP arrives over serial, the Arduino immediately kills motor PWM, aborts the remaining avoidance phases, and latches into STATE_PI_STOPPED. When PI_GO is subsequently issued, the car does not resume the aborted avoidance; it enters STATE_IDLE with a clean slate.

### Scenario 02: Sudden Physical Obstacle (Tier 2 Ultrasonic Override)

• Situation: While executing a high-level DRIVE command from the Pi, a low obstacle or reflective surface appears $22\,\text{cm}$ in front of the left sensor (within the calibrated $25\,\text{cm}$ threshold).

• Resolution: The Arduino firmware interrupts Tier 3 navigation locally within $10\,\text{ms}$ (debounced across 2 cycles), transitions to STATE_AVOIDING, executes an evasive pivot turn right at PWM 210 (or reverses $400\,\text{ms}$ at PWM 190 followed by turn if both sensors trigger), and notifies the Pi with [TELEM] AVOID RIGHT.

---

### 7. Comparative Benchmarking Summary

Our architecture has been benchmarked against major open-source mobile robotics frameworks:

Metric   |   Our Vehicle (Model-Car)   |   NVIDIA JetBot   |   DonkeyCar   |   OpenBot (Stanford/Intel)

Compute Type   |   2-Layer (Pi 5 + Arduino Uno)   |   Single-Board (Jetson Nano)   |   Single-Board (Pi 4 + PCA9685)   |   2-Layer (Smartphone + Arduino)

BOM Cost   |   $120–$150 (College Sourced)   |   $250–$300   |   $250–$300   |   $50 Body (Excludes $400+ Phone)

Motor PWM Stability   |   Hardware Timer (Zero Jitter)   |   Linux I2C / Kernel Jitter   |   I2C PWM Servo Driver   |   Hardware Timer on Arduino

Detection Stack   |   YOLOv10n (NMS-Free) + RDD2022   |   MobileNet-SSD / ResNet   |   Behavioral Cloning CNN   |   MobileNet-SSD (TFLite)

Pothole / Negative Obstacle   |   Native Deep Model & Laplacian CV   |   None (Blind to Potholes)   |   None   |   None

Follow-Me Capability   |   KCF + HSV Appearance Re-ID   |   None   |   None   |   MobileNet Person Tracking

Reflex Fallback   |   Autonomous Hard Real-Time FSM   |   None (Vision Only)   |   None   |   Sonar on Arduino Body

---

### 8. Directory & Module Map

model-car-capstone/

├── AGENTS.md                                    # Multi-agent roles, constraints, and contracts

├── README.md                                    # Repository overview and getting started

├── arduino/

│   ├── PROTOCOL.md                              # Authoritative serial command/telemetry contract

│   ├── lib/

│   │   ├── pins.h                               # Hardware pin assignments (Architecture v2 single source of truth)

│   │   └── protocol.h                           # Protocol constants, baud, timing thresholds

│   ├── motor_test/

│   │   ├── motor_test.ino                       # Legacy single-motor test (guarded)

│   │   └── dual_motor_speed_test.ino            # Verified working 4WD skid-steer obstacle avoidance & diagnostic suite

│   ├── dual_motor_speed_test/                   # Dedicated standalone Arduino IDE sketch folder

│   │   └── dual_motor_speed_test.ino

│   └── point_to_point_control/

│       ├── point_to_point_control.ino           # Production 3-tier FSM firmware

│       ├── pins.h                               # Local copy for Arduino IDE build compatibility

│       └── protocol.h                           # Local copy for Arduino IDE build compatibility

├── car/

│   ├── main.py                                  # Core perception-decision-actuation loop

│   ├── requirements.txt                         # Runtime dependencies (ultralytics, opencv, pyserial)

│   ├── control/

│   │   ├── motor_controller.py                  # Pi-side serial client and command sender

│   │   ├── navigation.py                        # Dead-reckoning and waypoint driving

│   │   └── pid.py                               # Proportional-Integral-Derivative steering controller

│   └── perception/

│       ├── config.py                            # Model paths, camera resolutions, serial port configs

│       ├── detector.py                          # YOLOv10n NCNN wrapper + classical CV pothole detector

│       ├── avoidance.py                         # Danger zone scoring engine (Left/Center/Right)

│       ├── tracker.py                           # KCF tracker + HSV color appearance re-ID

│       └── sensors.py                           # Serial telemetry parser and ultrasonic tracker

├── pothole_detection/                           # Dedicated Pothole & Road Defect Module

│   ├── CONTEXT.md                               # Pothole detection theory, architecture, and HUD specs

│   ├── run_detection.py                         # Directional swerve & perspective corridor inference engine

│   ├── train_pothole_model.py                   # YOLOv8n fine-tuning pipeline on RDD2022

│   ├── prepare_rdd_yolo.py                      # RDD2022 dataset conversion and augmentation script

│   ├── best.pt / rdd2022_yolov8n.pt             # Fine-tuned model weights (India D40 Pothole class)

│   ├── yolov10n.pt                              # Positive obstacle model weights

│   └── runs_rdd/custom_pothole_model/           # Empirical training metrics (PR/F1 curves, confusion matrix)

├── docs/

│   ├── architecture.md                          # THIS DOCUMENT (Canonical System Architecture)

│   ├── concerns_and_future_todos.md             # Edge-case analysis, calibration roadmap

│   ├── decisions/                               # Architecture Decision Records (ADRs)

│   │   └── 0001-usb-serial-vs-gpio-uart.md

│   ├── reports/                                 # Formal Capstone Benchmarking Reports (DOCX + MD)

│   └── scenarios/                               # 10 formal operational behavioral specifications

│       ├── 01_clear_path.md ... 10_rapid_pi_commands.md

│       └── README.md

└── infra/

├── SETUP.md                                 # Headless Pi 5 bringup, interfaces (CSI, I2C, Serial)

└── pi-image/                                # cloud-init Wi-Fi and headless provisioning configs

---

### 9. Architecture Decision Records (ADRs)

### ADR-0001: Transport Selection — USB-Serial vs. GPIO-UART

• Decision: Adopt USB-Serial (/dev/ttyUSB0 or /dev/ttyACM0) over Raspberry Pi 40-pin GPIO UART (/dev/ttyAMA0).

• Rationale:

1. GPIO UART requires disabling the Linux serial login shell and Bluetooth overlay (dtoverlay=disable-bt), introducing setup fragility across OS updates.

2. USB-Serial provides native opto-isolated buffering through the ATmega16U2/CH340 chip, protecting the Pi 5's delicate $3.3\,\text{V}$ unbuffered SoC GPIO pins from $5\,\text{V}$ Arduino logic levels and inductive transients.

3. Standard USB cables eliminate jumper wire disconnections during chassis road vibration.

### ADR-0002: Perception Inference Engine — Tencent NCNN over ONNX Runtime / PyTorch

• Decision: Export YOLO models to Tencent NCNN for production Pi 5 execution.

• Rationale:

1. NCNN is explicitly hand-optimized for ARM architectures with assembly-level NEON SIMD vectorization.

2. NCNN requires zero heavy Python framework dependencies (eliminating PyTorch and TorchScript overhead), drastically reducing memory footprint and boot times.

3. Eliminates memory fragmentation and thread contention with the camera ingest loop.

### ADR-0003: Microcontroller Evolution — Future Nano Migration

• Decision: Design all pin assignments and firmware for compatibility with the Arduino Nano (ATmega328P).

• Rationale: The Arduino Uno R3 and Arduino Nano share the identical ATmega328P microcontroller core, pin count, register mappings, and hardware timers. Migrating to the Nano in future revisions will reduce chassis footprint by $70\%$ with zero firmware modifications.

### ADR-0004: Hardware Pinout Realignment & IR Sensor Removal

• Decision: Re-assign front ultrasonic transceivers to Left = D7 (Trig) / D13 (Echo) and Right = D12 (Trig) / D4 (Echo), designate pins D2 and D3 as UNCONNECTED/RESERVED, and eliminate IR proximity sensors from the physical design.

• Rationale:

1. Physical tracing revealed pins D2 and D3 were completely empty on the physical chassis; phantom code references to IR sensors were legacy artifacts.

2. Ultrasonic wiring was physically matched to D7/D13 and D12/D4. D13 on Arduino Uno has an onboard LED that flickers during echo pulses, but provides flawless $1\,\mu\text{s}$ timing resolution.

3. Freeing D2 (INT0) and D3 (INT1) preserves critical hardware interrupt pins for future rotary wheel encoder integration without requiring breadboard multiplexing.

### ADR-0005: TT Motor Polarity Inversion & L298N BJT Torque Compensation

• Decision: Enforce firmware-level direction inversion (INVERT_MOTOR_POLARITY = true / INVERT_DIR = true) and calibrate operating PWM levels to $190\text{–}210$ (replacing legacy PWM 120/150).

• Rationale:

1. Inverted chassis wiring caused reverse travel on forward logic. Modifying wiring violates the borrowed lab hardware constraint; inverting logic bits in firmware completely solves the problem with zero physical risk.

2. The L298N's internal bipolar transistors induce a $\approx 2.0\,\text{V}$ drop. From a 2S Li-Ion pack ($7.4\,\text{V}$), only $\sim 5.4\,\text{V}$ reaches the motors. A PWM of 120 provided insufficient torque to break static friction, causing chassis stalls. Operating at PWM 200 (forward) and 210 (pivot turns) delivers reliable breakaway torque and skid-steer maneuvering.

### ADR-0006: Vision Sensor Transition — OmniVision OV5647 5MP Adoption

• Decision: Adopt the OmniVision OV5647 5MP Camera Module (Rev 1.3) via a 22-pin to 15-pin FPC adapter cable, replacing the cracked Raspberry Pi Global Shutter camera.

• Rationale:

1. Physical glass damage to the borrowed Global Shutter module rendered it non-viable. The OV5647 is widely available, cost-effective, and supported natively by the Raspberry Pi 5 PiSP kernel driver.

2. The fixed/manual focus lens provides immediate optical sharpness without the heavy blur caused by un-tuned C/CS manual lenses.

3. The optical horizontal FOV shifts from $120^\circ$ (wide angle) to $\approx 53.5^\circ$ (standard $3.6\,\text{mm}$ lens). Perception and zone-scoring logic in car/perception/config.py have been aligned to CAMERA_HFOV_DEG = 53.5.

4. Diagnostic bench viewing is decoupled from high-latency Pi Connect desktop screen-sharing via a dedicated local HTTP MJPEG server ([tools/stream.py](../tools/stream.py)). Autonomous driving pipelines bypass streaming entirely, streaming frames in-memory via picamera2.capture_array().

# APPENDIX B — ARCHITECTURAL CONCERNS AND FUTURE WORK REGISTER

Source-status note: this is a project issue and decision register. Proposed mitigations are not proof of implementation or successful testing. Several entries identify unresolved calibration, watchdog, command-rate, and hardware tasks.

## Architectural Concerns & Future Roadmap (TODOs)

Document Status: ACTIVE / AUTHORITATIVE

Date: 2026-09-29

Source: Architecture Brainstorming Review (docs/architecture.md Stress Test)

Owners: Krishna (Lead / Hardware / Firmware), Aditya (Hardware / Chassis), Parth (Software / Perception)

---

### 1. Executive Summary

During the systematic brainstorming and stress-testing of [docs/architecture.md](file:///e:/OneDrive%20-%20Shri%20Vile%20Parle%20Kelavani%20Mandal/Capstone/model/docs/architecture.md), nine key architectural concerns, edge cases, and design gaps were identified. Each concern was analyzed with engineering trade-offs, and clear resolutions or future hardware/integration TODOs were established.

This document serves as the permanent record of these design concerns, their mitigations, and the master TODO list for bench and track testing.

---

### 2. Architectural Design Concerns & Decisions

### Concern 1: Avoidance Re-Trigger Loop during Resume Phase

• Context: In Tier 2 ultrasonic avoidance, the state machine executes three phases: Phase 1 (Reverse 400ms) → Phase 2 (Skid-turn 500ms away from obstacle) → Phase 3 (Resume forward 2000ms at saved drive speed) → IDLE.

• The Problem: Ultrasonics poll continuously every 100ms. If the 500ms skid-turn was insufficient to fully clear a wide barrier or wall corner, the ultrasonic sensors will immediately register < 20cm during Phase 3, triggering avoidance anew. This creates an infinite Reverse → Turn → Resume → Detect → Reverse... loop.

• Resolution:

- Introduce an AVOID_COOLDOWN_MS constant (default 500ms) in firmware.

- After Phase 2 (Turn) completes and Phase 3 (Resume) initiates, ultrasonic obstacle re-triggering is suppressed for AVOID_COOLDOWN_MS.

- This guarantees the car drives clear of the obstacle boundary before reflex avoidance can re-arm.

---

### Concern 2: TURN Angle-to-Duration Linear Mapping is Uncalibrated

• Context: The serial protocol specifies TURN <angle> <speed>. In the initial Arduino code, turn duration was computed via an arbitrary linear formula: duration_ms = abs(angle) * 10.

• The Problem: On a 4WD acrylic skid-steer chassis with DC TT gearmotors, physical rotational degrees are non-linear and heavily influenced by ground friction, tire grip, chassis weight distribution, and 2S battery discharge voltage. abs(angle) * 10 is an unverified placeholder.

• Resolution & Future TODO:

- Keep TURN <angle> <speed> command format in PROTOCOL.md (no API breakage).

- Add a named calibration constant TURN_MS_PER_DEGREE in [arduino/lib/protocol.h](file:///e:/OneDrive%20-%20Shri%20Vile%20Parle%20Kelavani%20Mandal/Capstone/model/arduino/lib/protocol.h).

- TODO: Calibrate this parameter empirically on real hardware across carpet and tile surfaces once the 4-wire PWM migration is complete.

---

### Concern 3: Follow-Me Steering Disruption During Tier 2 Avoidance & IMU Heading Recovery

• Context: In Follow-Me mode, the Pi 5 vision loop sends continuous steering corrections (~30Hz) toward the tracked person. When an obstacle enters the 20cm zone, Arduino reflex Tier 2 avoidance overrides Tier 3 motor commands for ~2.9 seconds.

• The Problem:

1. The Arduino silently drops Tier 3 commands while in STATE_AVOIDING (current design does not queue commands).

2. After completing avoidance, the car points in a completely new heading, and the Pi's PID loop would suddenly be facing the wrong direction without awareness of how much the car rotated.

• Resolution & Future TODO:

- Tier 3 commands sent during avoidance will continue to be dropped (clean replace semantics; no queue).

- Pi 5 must parse [TELEM] AVOID <DIR> and pause its PID tracker while avoidance is in progress.

- TODO (IMU Heading Compensation): The car hardware bill of materials includes an ICM20948 9-DOF IMU over I2C to the Pi. When avoidance finishes ([TELEM] DONE AVOID), the Pi reads the IMU heading delta accumulated during the maneuver and applies a heading correction to immediately re-orient toward the target before resuming visual KCF tracking.

---

### Concern 4: Pi 5 Crash / Serial Disconnect & Watchdog Fail-Safe

• Context: The two-layer architecture separates real-time motor control (Uno) from heavy neural inference (Pi 5).

• The Problem: If the Pi 5 freezes under heavy YOLOv10n load, crashes, or the USB serial cable disconnects, what prevents the car from running away or getting stuck in an undefined state?

• Resolution:

- All Tier 3 motion commands (DRIVE, REV, TURN) are strictly bounded-duration; the car naturally halts when the command duration expires.

- Add a dedicated Serial Watchdog in the Arduino firmware:

- Variable lastPiCommandTime updated on any valid command or PING.

- If millis() - lastPiCommandTime > WATCHDOG_TIMEOUT_MS (default 5000ms):

1. Immediate transition to STATE_IDLE (all PWM set to 0).

2. Periodic broadcast of telemetry alert: [TELEM] HEARTBEAT_TIMEOUT.

- When the Pi recovers, it detects HEARTBEAT_TIMEOUT in the telemetry stream and resets its communication handshake cleanly.

---

### Concern 5: Continuous 30Hz PID Steering vs. Discrete Timed `TURN` Commands

• Context: The PID controller in car/control/pid.py computes an angular error at the camera frame rate (~30Hz, every ~33ms).

• The Problem: Sending TURN <angle> <speed> with a duration of 200ms every 33ms means every incoming command resets the Arduino's turning timer before it ever completes. This can cause jerky, stuttered motion where motor states restart continuously.

• Resolution & Future TODO:

- Acknowledge this impedance mismatch between continuous control and discrete timed commands.

- TODO: Evaluate three candidates during on-chassis integration testing:

1. Option A (Differential Drive): Add DRIVE_DIFF <left_pwm> <right_pwm> <duration_ms> so PID directly modulates wheel speeds.

2. Option B (Micro-Pulses): PID issues short sub-cycle turns (e.g., 30–40ms duration) that naturally match the frame interval.

3. Option C (Continuous Steer Command): Introduce a dedicated STEER <left_speed> <right_speed> command with short watchdog timeouts designed specifically for streaming PID setpoints.

---

### Concern 6: Protocol Header Sync & Missing Constants

• Context: The architecture spec guarantees that all avoidance parameters are configurable via named constants in protocol.h.

• The Problem: AVOID_RESUME_MS and AVOID_COOLDOWN_MS were specified in documentation but missing from [arduino/lib/protocol.h](file:///e:/OneDrive%20-%20Shri%20Vile%20Parle%20Kelavani%20Mandal/Capstone/model/arduino/lib/protocol.h).

• Resolution:

- Update protocol.h to declare:

- #define AVOID_REVERSE_MS 400

- #define AVOID_TURN_MS 500

- #define AVOID_RESUME_MS 2000

- #define AVOID_COOLDOWN_MS 500

- #define WATCHDOG_TIMEOUT_MS 5000

---

### Concern 7: Follow-Me Re-ID False Match Risks with Similar Clothing

• Context: The owner profile uses HSV color histograms (top 40% upper body, bottom 40% lower body, aspect ratio) for fast re-identification when KCF loses lock.

• The Problem: If a bystander wears clothing of similar color hue, histogram correlation (cv2.compareHist) may score above threshold, causing the car to follow the wrong individual.

• Resolution:

- This is an accepted design constraint to keep latency within the Pi 5's 30 FPS budget (neural Re-ID models like OSNet are too computationally heavy alongside YOLOv10n).

- Configurable similarity threshold REID_SIMILARITY_THRESHOLD (baseline 0.80).

- If no detected bounding box exceeds the correlation threshold, the car refuses to lock on, halts in IDLE, and waits for the genuine owner to re-enter view.

---

### Concern 8: Autonomous Mode Waypoint Definition

• Context: The dual-mode architecture specifies Autonomous Navigation as Mode 1 and Follow-Me as Mode 2.

• The Problem: While Follow-Me has a full pipeline (lock-on → KCF → PID → re-ID), Autonomous Navigation is currently defined as an open abstraction ("drive forward / waypoints and avoid obstacles").

• Resolution & Future TODO:

- Follow-Me mode is the primary deliverable for the capstone demonstration.

- Autonomous navigation with coordinate waypoints, dead-reckoning integration, and path planning is formally classified as Future Scope (Phase 2).

- For baseline autonomous testing, the car will operate in "reactive cruise" mode (drive forward at set speed, obstacle avoidance reflexes enabled).

---

### Concern 9: Serial Buffer Overflow Risk Under Blocking Ultrasonic Reads

• Context: The Arduino Uno has a 64-byte hardware UART receive buffer. At 115200 baud, a 16-byte command arrives in ~1.4ms.

• The Problem: Arduino readUltrasonicCm() uses pulseIn() with a 30,000µs (30ms) timeout. If both left and right sensors timeout (no echo received), the Arduino blocks for up to ~60ms. During this interval, multiple rapid PID commands from the Pi could overflow the 64-byte UART buffer.

• Resolution:

- Noted design observation.

- If serial buffer overflow is observed in testing:

1. Reduce US_TIMEOUT_US from 30000µs to 12000µs (sufficient for ~200cm range; beyond obstacle threshold).

2. Interleave ultrasonic polling (ping left sensor on odd ticks, right sensor on even ticks).

---

### 3. Master Future TODO List

Category   |   Item   |   Description   |   Priority   |   Owner

Hardware   |   4-Wire PWM Migration   |   Move ENA/ENB jumpers to D5/D6; move Echo Left/Right to A0/A1.   |   🔴 Blocker   |   Krishna

Hardware   |   Scrap IR Sensors   |   Remove 2× IR sensors to reduce Berg bus clutter and free D2/D3 digital pins.   |   🟡 High   |   Aditya

Hardware   |   Rear Ultrasonic Evaluation   |   Mount 1× HC-SR04P facing backward using freed D2/D3 pins to eliminate reverse blind spot.   |   🟢 Medium   |   Aditya / Krishna

Hardware   |   Nano Migration   |   Swap Uno for Arduino Nano (identical ATmega328P pinout, smaller footprint).   |   🔵 Future   |   Krishna

Firmware   |   Resolve Merge Conflict   |   Fix <<<<<<< HEAD merge conflict markers at line 398 in point_to_point_control.ino.   |   🔴 Blocker   |   firmware-agent

Firmware   |   Add Watchdog & Constants   |   Implement WATCHDOG_TIMEOUT_MS, AVOID_COOLDOWN_MS, and AVOID_RESUME_MS in protocol.h and .ino.   |   🟡 High   |   firmware-agent

Firmware   |   Calibrate Turn Constant   |   Empirically calibrate TURN_MS_PER_DEGREE on physical chassis across battery voltage levels.   |   🟡 High   |   firmware-agent

Perception   |   Align Telemetry Parser   |   Update sensors.py and config.py from stale 3-sonar center format to 2-sensor US LEFT=xx RIGHT=yy.   |   🔴 Blocker   |   perception-agent

Perception   |   Calibrate Distance Bboxes   |   Test camera bounding box height % (15% far, 40% close) against physical meter distances.   |   🟡 High   |   perception-agent

Perception   |   Implement Owner Profile   |   Build HSV upper/lower body feature extraction and correlation matcher in tracker.py.   |   🟡 High   |   perception-agent

Integration   |   PID Steering vs TURN Model   |   Benchmark continuous PID setpoints on real chassis; decide between Differential DRIVE vs Micro-TURNs.   |   🟡 High   |   integration-agent

Integration   |   IMU Post-Avoidance Recovery   |   Integrate ICM20948 I2C telemetry on Pi to compute heading delta after Tier 2 avoidance.   |   🟢 Medium   |   integration-agent

Integration   |   Autonomous Waypoint FSM   |   Define waypoint data structure and dead-reckoning trajectory planner for autonomous mode.   |   🔵 Future   |   integration-agent

---

### 4. Cross-References

• System Architecture: [architecture.md](architecture.md)

• Serial Protocol Definition: [../arduino/PROTOCOL.md](../arduino/PROTOCOL.md)

• Hardware Wiring Baseline: [../arduino_rf_robot_pin_configuration_wiring_guide.md](../arduino_rf_robot_pin_configuration_wiring_guide.md)

• Behavioral Scenarios: [scenarios/README.md](scenarios/README.md)

# APPENDIX C — SERIAL PROTOCOL SPECIFICATION

This specification is the repository’s stated source of truth. Confirm the uploaded firmware and Pi-side bridge implement the same contract.

## Serial Protocol Specification (Source of Truth)

Layer Contract: Raspberry Pi 5 ↔ Arduino Uno (ATmega328P)

Baud Rate: 115200 baud, 8-N-1

Line Ending: \n (newline, ASCII 0x0A)

Transport: Swappable — USB-Serial (/dev/ttyUSB0 or /dev/ttyACM0) or GPIO-UART (/dev/ttyAMA0).

---

### 1. Command Priority Model

Three tiers, highest priority wins:

Tier   |   Authority   |   Commands   |   Override behavior

1 — Pi Camera Latch   |   Raspberry Pi 5 (camera + YOLO)   |   PI_STOP, PI_GO, PI_AVOID   |   Absolute. Freezes all motor output. Only PI_GO releases.

2 — Local Ultrasonic   |   Arduino Uno (2× HC-SR04P)   |   (automatic — no command)   |   Triggers autonomous avoidance. Preempted by Tier 1.

3 — Normal Driving   |   Pi 5 or Serial Monitor   |   DRIVE, REV, TURN, STOP, PING   |   Standard timed actions. Preempted by Tier 1 and 2.

---

### 2. Commands (Pi 5 → Arduino Uno)

Commands are sent as ASCII strings terminated with \n.

### Tier 1 — Pi Camera Latch

Command   |   Format   |   Example   |   Description

PI_STOP   |   PI_STOP   |   PI_STOP   |   Latched emergency stop from camera algorithm (obstacle / negative obstacle). Freezes all motors. Only PI_GO releases. Overrides everything, including mid-avoidance.

PI_GO   |   PI_GO   |   PI_GO   |   Release Pi latch. Returns to STATE_IDLE. Ignored if not in PI_STOPPED state.

PI_AVOID   |   PI_AVOID <LEFT\   |   RIGHT>   |   PI_AVOID LEFT   |   Directional hint for next avoidance maneuver. Stored and consumed once. If set when ultrasonic avoidance triggers, car turns this direction instead of deciding locally.

### Tier 3 — Normal Driving

Command   |   Format   |   Example   |   Description

DRIVE   |   DRIVE <speed> <duration_ms>   |   DRIVE 180 1500   |   Drive forward at PWM <speed> (0–255) for <duration_ms> ms

REVERSE   |   REV <speed> <duration_ms>   |   REV 180 1000   |   Drive reverse at PWM <speed> (0–255) for <duration_ms> ms

TURN   |   TURN <angle_deg> <speed>   |   TURN 45 150   |   Skid-steer turn by <angle_deg> (-180 to +180; positive = right, negative = left)

STOP   |   STOP   |   STOP   |   Immediate non-latched stop (goes to IDLE; does NOT lock out further commands)

PING   |   PING   |   PING   |   Liveness check

> Note: All Tier 3 commands are blocked while PI_STOPPED is active. Arduino responds with [ACK] BLOCKED (PI_STOPPED).

---

### 3. Telemetry (Arduino Uno → Pi 5)

Arduino broadcasts or responds with single-line telemetry events prefixed by [TELEM] or [ACK].

### Acknowledgments

Message   |   Example   |   Description

ACK   |   [ACK] DRIVE 180 1500   |   Command accepted and executed/queued

PONG   |   [ACK] PONG   |   Heartbeat response to PING

BLOCKED   |   [ACK] BLOCKED (PI_STOPPED)   |   Command rejected — Pi latch is active

### State & Lifecycle

Message   |   Example   |   Description

READY   |   [TELEM] READY Uno-ATmega328P   |   Broadcast once bootloader finishes in setup()

STATE   |   [TELEM] STATE=DRIVING SPEED=180   |   Current state machine status

DONE   |   [TELEM] DONE DRIVE   |   Timed drive/turn/rev action finished

### Ultrasonic & Avoidance

Message   |   Example   |   Description

US   |   [TELEM] US LEFT=15.3 RIGHT=42.7   |   Periodic ultrasonic distance readings (every ~100ms)

AVOID   |   [TELEM] AVOID RIGHT   |   Avoidance maneuver started, turning in stated direction

DONE AVOID   |   [TELEM] DONE AVOID   |   Avoidance maneuver complete, resuming forward

PI_STOPPED   |   [TELEM] STATE=PI_STOPPED   |   Acknowledges Pi latch is active

---

### 4. State Machine

States:

IDLE         — Motors off, awaiting commands

DRIVING      — Forward at PWM speed for timed duration

REVERSING    — Reverse at PWM speed for timed duration

TURNING      — Skid-steer turn for timed duration

AVOIDING     — Autonomous ultrasonic avoidance (reverse → turn → resume)

PI_STOPPED   — Latched stop from Pi camera (frozen until PI_GO)

Transitions:

IDLE ──[DRIVE/REV/TURN]──► DRIVING/REVERSING/TURNING

DRIVING/REVERSING/TURNING ──[timer expires]──► IDLE

DRIVING/REVERSING/TURNING ──[ultrasonic < 20cm]──► AVOIDING

AVOIDING ──[maneuver complete]──► DRIVING (resume)

ANY STATE ──[PI_STOP]──► PI_STOPPED

PI_STOPPED ──[PI_GO]──► IDLE

ANY STATE ──[STOP]──► IDLE

---

### 5. Conventions

• Both left motors and both right motors are paired channel-wise on a single L298N driver board (skid-steer mode).

• Pin mappings are centralized in arduino/lib/pins.h.

• Ultrasonic and avoidance timing constants are in arduino/lib/protocol.h.

• Any modification to this protocol must originate from firmware-agent and be mirrored in car/control/motor_controller.py.

# APPENDIX D — RASPBERRY PI HEADLESS SETUP NOTES

These steps describe intended provisioning. Verify the actual Pi image and interface configuration on the deployed device.

## Raspberry Pi 5 Headless Bring-Up & Setup Guide

Owned by: network-agent

Target Device: Raspberry Pi 5 (8GB)

Hostnames:

• Krishna: pi5-krishna

• Aditya: pi5-aditya

• Parth: pi5-parth

---

### 1. Cloud-Init One-Shot Flashing

> [!WARNING]

> Cloud-init files (network-config and user-data) are evaluated only once on first boot. If Wi-Fi fails to connect, edit the files on the boot partition of the SD card/NVMe and reflash or recreate the configuration.

• Flash Raspberry Pi OS (64-bit Lite or Desktop) using Raspberry Pi Imager.

• In the bootfs partition, copy:

- infra/pi-image/network-config → /boot/firmware/network-config (or root of boot partition)

- infra/pi-image/user-data → /boot/firmware/user-data

• Eject safely and insert into the Raspberry Pi 5.

• Power up with the dedicated boAt EnergyShroom PB400 power bank.

---

### 2. Interface Enablement via `raspi-config`

Once connected via SSH (ssh pi@pi5-krishna.local):

Run:

sudo raspi-config

Enable the following interfaces:

• Camera (CSI): Interface Options → Camera → Enable

• I2C: Interface Options → I2C → Enable (required for ICM20948 IMU)

• Serial Port:

- Interface Options → Serial Port

- Would you like a login shell to be accessible over serial? → NO (CRITICAL: frees UART for Arduino Uno link)

- Would you like the serial port hardware to be enabled? → YES

Reboot to apply changes:

sudo reboot

---

### 3. SSH and Raspberry Pi Connect

• SSH: Enabled by default via user-data.

• Pi Connect: Optional browser-based remote access:

sudo apt install rpi-connect

rpi-connect on

rpi-connect signin

# APPENDIX E — POTHOLE PERCEPTION DESIGN NOTES

This source contains alternative hardware and pipeline descriptions from earlier work. It mentions host hardware, sensors, models, and performance claims that do not necessarily match the current Pi 5 + Uno runtime. Use it as historical design context only; the current code and hardware determine the as-built report.

## CONTEXT.md — Negative Obstacle & Pothole Avoidance System

================================================================================

> Project: Autonomous RC Car Capstone Project — Negative Obstacle Perception & Reactive Avoidance

> Target Benchmark: Kaggle India Road Damage Dataset (RDD2022 — D40 Potholes & Cracks)

> Core Vision Model: YOLOv10n (NMS-Free Dual-Head Real-Time Detector, 3.2M parameters)

> Deployment Weight: [best.pt](file:///e:/Pothole%20detection/best.pt) (5.75 MB — 50 Epochs Fine-Tuned on RTX 3050 Laptop GPU)

> Primary Script: [run_detection.py](file:///e:/Pothole%20detection/run_detection.py)

> Training Engine: [train_pothole_model.py](file:///e:/Pothole%20detection/train_pothole_model.py)

> Active Status: Fully functional, verified on 1080p POV & front dashcam test footage.

---

### 1. System Motivation & Problem Formulation

### 1.1 The Critical Dilemma: Positive vs. Negative Obstacles

In mobile robotics and autonomous vehicle design, obstacles fall into two fundamental physical classes:

• Positive Obstacles: Structures that rise vertically above the ground plane (walls, pillars, curbs, other vehicles, pedestrians, debris).

• Negative Obstacles: Geometric depressions, voids, or cavities that drop below the drivable surface plane (potholes, road trenches, open manholes, eroded asphalt shoulders, missing bridge expansion slabs).

[ HORIZONTAL ACTIVE SENSORS (Ultrasonic / 2D LiDAR) ]

│

Sound / Laser Ray ──────┼─────────────────────────► (Travels straight into air)

▼                          (NO ECHO / NO REFLECTION)

┌───────────────┐

│ AUTONOMOUS RC │═════════════════════════════╗

│  CAR CHASSIS  │                             ║

└───────────────┘                             ▼

│

[ CAMERA ] ────────────────────────────────► ╲    POTHOLE    ╱ ──► [ CAMERA MUST DETECT ]

(Down/Ahead)                                 ╲  DEPRESSION ╱

### 1.2 Why Active Ranging Sensors (Ultrasonic & LiDAR) Fail

• Ray Propagation Geometry: Standard automotive ultrasonic rangefinders (e.g., HC-SR04P) and planar 2D LiDAR units emit pulses along a horizontal cone. When confronted with a pothole, the emitted sound or optical beam propagates straight across the opening into the open air beyond. No surface echo reflects back, causing the sensor to report a completely clear path.

• Failure Consequence: An autonomous RC car relying exclusively on ultrasonic sensors will drive directly into potholes at cruise speed (PWM 180-255), causing violent wheel entrapment, steering tie-rod fracture, chassis bottoming, and motor stall currents that trip battery protection circuits.

• The Monocular Vision Imperative: Downward-and-forward angled optical perception is the only cost-effective sensor modality capable of identifying negative road surface defects early enough to execute safe steering evasion maneuvers.

---

### 2. End-to-End System Architecture

The project employs a hierarchical two-tier distributed architecture dividing heavy computer vision perception from deterministic low-level actuator control:

┌─────────────────────────────────────────────────────────────────────────────────────────────┐

│                           TIER 1: HIGH-LEVEL VISION & DECISION BRAIN                        │

│                   (Host Laptop / NVIDIA Jetson Nano / Raspberry Pi 5)                       │

│                                                                                             │

│   ┌────────────────────────┐      ┌─────────────────────────┐      ┌────────────────────┐   │

│   │ Monocular Video Feed   │ ───► │ Perspective ROI Engine  │ ───► │ YOLOv10n Semantic  │   │

│   │ (720p/1080p @ 30 FPS)  │      │ (Auto Dashcam / Cabin)  │      │ Inference Engine   │   │

│   └────────────────────────┘      └─────────────────────────┘      └─────────┬──────────┘   │

│                                                                              │              │

│                                                                              ▼              │

│   ┌────────────────────────┐      ┌─────────────────────────┐      ┌────────────────────┐   │

│   │ HUD Telemetry Overlay  │ ◄─── │ Avoidance Matrix Engine │ ◄─── │ Spatial Corridor   │   │

│   │ (Visual Feedback / MP4)│      │ (Risk Evaluation)       │      │ Risk Calculator    │   │

│   └────────────────────────┘      └───────────┬─────────────┘      │ (Left/Center/Right)│   │

│                                               │                    └────────────────────┘   │

└───────────────────────────────────────────────┼─────────────────────────────────────────────┘

│ UART Serial (115200 Baud)

▼

┌─────────────────────────────────────────────────────────────────────────────────────────────┐

│                      TIER 2: REAL-TIME MOTION & REFLEX CONTROLLER                           │

│                                 (Arduino Uno / ESP32)                                       │

│                                                                                             │

│   ┌────────────────────────┐      ┌─────────────────────────┐      ┌────────────────────┐   │

│   │ Command Serial Parser  │ ───► │ State Machine & Arbiter │ ───► │ Motor & Servo PWM  │   │

│   │ (PI_STOP, TURN, DRIVE) │      │ (Overrides & Failsafes) │      │ Drivers (L298N)    │   │

│   └────────────────────────┘      └───────────▲─────────────┘      └─────────┬──────────┘   │

│                                               │                              │              │

│                                   ┌───────────┴─────────────┐                ▼              │

│                                   │ HC-SR04P Ultrasonics    │      ┌────────────────────┐   │

│                                   │ (Emergency Wall Reflex) │      │ Steering Servo &   │   │

│                                   └─────────────────────────┘      │ Drive Wheels (DC)  │   │

│                                                                    └────────────────────┘   │

└─────────────────────────────────────────────────────────────────────────────────────────────┘

### 2.1 Hardware Specifications

• Host Perception Processor:

* Development / Validation: Intel/AMD Multi-core CPU + NVIDIA GeForce RTX 3050 Laptop GPU (4GB VRAM, CUDA 12.6, TensorRT / FP16 support).

* Target Embedded Vehicle Unit: Raspberry Pi 5 (8GB) or NVIDIA Jetson Nano / Orin Nano.

• Microcontroller Unit (MCU):

* Arduino Nano / Uno (ATmega328P) or ESP32-WROOM-32.

* Clock: 16 MHz (ATmega) / 240 MHz (ESP32).

* Logic Level: 5V (Arduino) or 3.3V (ESP32) with bi-directional logic level converter on UART lines.

• Drive Train & Actuators:

* Steering: SG90 / MG996R Metal Gear Servo ($-60^\circ$ full left to $+60^\circ$ full right; $0^\circ$ center trim).

* Traction: Dual Brushed DC Motors with 1:48 reduction gearbox.

* Motor Driver: L298N Dual H-Bridge or TB6612FNG MOSFET Driver.

• Sensors:

* Vision: Front-facing wide-angle HD Camera (1080p, 120° FOV, mounted 12 cm above ground).

* Ultrasonics: Dual HC-SR04P rangefinders mounted laterally at 45° angles for perimeter clearance.

### 2.2 Serial Communication Protocol (Pi/Host $\leftrightarrow$ Arduino)

Defined in [negative_obstacle_detection/05_pothole_ahead.md](file:///e:/Pothole%20detection/negative_obstacle_detection/05_pothole_ahead.md):

• Baud Rate: 115200 bps, 8 data bits, no parity, 1 stop bit (8-N-1).

• Format: Plaintext ASCII terminated with newline (\n).

Time Offset   |   Direction   |   Message String   |   Functional Purpose

0 ms   |   MCU $\rightarrow$ Host   |   [TELEM] STATE=DRIVING SPEED=180   |   Telemetry heartbeat from low-level controller.

120 ms   |   Host $\rightarrow$ MCU   |   PI_STOP   |   Vision detects immediate pothole hazard in center corridor.

125 ms   |   MCU $\rightarrow$ Host   |   [ACK] PI_STOP   |   MCU cuts motor PWM to 0; car halts safely.

250 ms   |   Host $\rightarrow$ MCU   |   PI_AVOID LEFT   |   Host registers avoidance route suggestion.

450 ms   |   Host $\rightarrow$ MCU   |   PI_GO   |   Host unlatches emergency stop state.

550 ms   |   Host $\rightarrow$ MCU   |   TURN -30 150   |   Steer wheels $30^\circ$ left at moderate drive speed.

1200 ms   |   Host $\rightarrow$ MCU   |   DRIVE 150 2000   |   Cruise past the pothole cavity for 2000 ms.

3250 ms   |   Host $\rightarrow$ MCU   |   TURN 30 150   |   Counter-steer $30^\circ$ right to re-align vehicle with road center.

3900 ms   |   Host $\rightarrow$ MCU   |   DRIVE 180 5000   |   Resume normal cruising speed (PWM 180).

---

### 3. Dataset Engineering: RDD2022 India (`D40`)

The detection model was trained and conditioned on the official Road Damage Dataset 2022 (RDD2022) India subset.

### 3.1 Raw Dataset Architecture

• Location: [negative_obstacle_detection/train/](file:///e:/Pothole%20detection/negative_obstacle_detection/train)

• Total Raw Images: 1,530 high-resolution camera frames (India_XXXXXX.jpg) captured from vehicles on diverse Indian municipal and rural roads under varied lighting.

• Annotation Standard: Pascal VOC XML files (India_XXXXXX.xml) containing bounding box coordinates [xmin, ymin, xmax, ymax] and categorical class names.

### 3.2 Automated VOC-to-YOLO Conversion Pipeline

Utility: [negative_obstacle_detection/prepare_rdd_yolo.py](file:///e:/Pothole%20detection/negative_obstacle_detection/prepare_rdd_yolo.py)

Target Destination: [negative_obstacle_detection/rdd_yolo_dataset/](file:///e:/Pothole%20detection/negative_obstacle_detection/rdd_yolo_dataset)

• Mathematical Coordinate Normalization:

$$x_{\text{center}} = \frac{x_{\min} + x_{\max}}{2 \cdot W}, \quad y_{\text{center}} = \frac{y_{\min} + y_{\max}}{2 \cdot H}$$

$$w = \frac{x_{\max} - x_{\min}}{W}, \quad h = \frac{y_{\max} - y_{\min}}{H}$$

• Class Mapping & Instance Distribution:

* Class 0: pothole_D40 $\rightarrow$ 3,187 labeled instances (Primary target of interest)

* Class 1: longitudinal_crack_D00 $\rightarrow$ 471 labeled instances

* Class 2: transverse_crack_D10 $\rightarrow$ 23 labeled instances

* Class 3: alligator_crack_D20 $\rightarrow$ 645 labeled instances

* Class 4: marking_defect_D44 $\rightarrow$ 151 labeled instances

• Train/Validation Split:

* Training Set: 80% (1,224 images)

* Validation Set: 20% (306 images)

• Ultralytics Config: [negative_obstacle_detection/rdd_yolo_dataset/rdd_dataset.yaml](file:///e:/Pothole%20detection/negative_obstacle_detection/rdd_yolo_dataset/rdd_dataset.yaml)

---

### 4. Deep Learning Model & 50-Epoch GPU Training

### 4.1 Architecture Selection: YOLOv10 Nano (`yolov10n`)

• NMS-Free Dual Label Assignment: Conventional object detectors (YOLOv5, YOLOv8) depend on Non-Maximum Suppression (NMS) during inference to filter redundant bounding boxes. NMS has $O(N^2)$ worst-case complexity and introduces unpredictable latency jitter. YOLOv10 employs dual label assignment during training (one-to-many for representation learning, one-to-one for direct prediction), eliminating NMS completely.

• Edge Compactness: Total parameters: 3.2 Million; Model weight size: 5.75 MB. This ensures ultra-fast forward passes on edge NPUs and CPU fallback modes.

### 4.2 Training Setup & Hardware Configuration

• Script: [train_pothole_model.py](file:///e:/Pothole%20detection/train_pothole_model.py)

• Compute Engine: NVIDIA GeForce RTX 3050 Laptop GPU (4GB GDDR6 VRAM).

• Software Stack: Python 3.13, PyTorch with CUDA 12.6 wheels, Ultralytics 8.3+.

• Hyperparameters:

* epochs: 50

* batch: 8 (VRAM footprint ~2.1 GB; guarantees zero OOM failures on 4GB GPUs)

* imgsz: 640

* amp: True (Mixed precision FP16 for acceleration)

* device: 0 (CUDA GPU)

* workers: 4

### 4.3 Training Progression & Convergence Milestones

Extracted directly from [runs_rdd/custom_pothole_model/results.csv](file:///e:/Pothole%20detection/runs_rdd/custom_pothole_model/results.csv):

Epoch   |   Cumulative Time (s)   |   Train Box Loss   |   Train Cls Loss   |   Precision   |   Recall   |   mAP@50   |   mAP@50-95   |   Val Box Loss   |   Val Cls Loss

1   |   44.4s   |   2.018   |   6.068   |   0.841   |   0.032   |   0.026   |   0.008   |   1.968   |   6.725

10   |   289.9s   |   1.958   |   3.003   |   0.235   |   0.128   |   0.108   |   0.042   |   1.902   |   3.059

20   |   588.0s   |   1.887   |   2.648   |   0.287   |   0.220   |   0.160   |   0.062   |   1.921   |   2.694

30   |   891.3s   |   1.841   |   2.423   |   0.299   |   0.212   |   0.197   |   0.091   |   1.936   |   2.514

40   |   1194.5s   |   1.794   |   2.181   |   0.534   |   0.238   |   0.232   |   0.101   |   1.941   |   2.391

50   |   1495.3s (~24.9m)   |   1.711   |   1.915   |   0.540   |   0.271   |   0.252   |   0.114   |   1.923   |   2.336

• Analysis: Classification loss decreased dramatically from $6.07 \rightarrow 1.91$ ($>68\%$ reduction). Mean Average Precision at IoU 0.50 (mAP50) expanded by nearly $10\times$ from $0.026 \rightarrow 0.252$.

• Final Weights: Deployed to [best.pt](file:///e:/Pothole%20detection/best.pt) in the workspace root.

---

### 5. Computer Vision & Perspective Geometry Engineering

### 5.1 The Interior Cabin & A-Pillar Failure Mode

In preliminary iterations, an experimental classical Laplacian edge detector (cv_detect_surface_defects) was coupled with YOLO:

• The Root Cause: Classical edge detectors compute second-order intensity spatial derivatives ($\nabla^2 I$). In vehicle interior POV footage (such as [test_video.mp4](file:///e:/Pothole%20detection/test_video.mp4)), the dark automotive interior structure (the right A-pillar, steering wheel, and rearview mirror) creates an intense contrast boundary against the bright sky.

• The Failure: The classical filter flagged the right A-pillar as a massive road cavity on every single frame, corrupting the steering matrix and forcing permanent SWERVE LEFT commands.

• The Resolution:

1. Completely disabled and removed classical Laplacian edge filtering.

2. Shifted 100% of detection responsibility to the fine-tuned semantic YOLOv10n neural network.

3. Engineered an intelligent Auto-Perspective ROI Engine in [run_detection.py](file:///e:/Pothole%20detection/run_detection.py).

### 5.2 Dual Auto-Perspective ROI Engine

The visual pipeline detects whether the input is a low-angle front bumper/dashcam view or an in-cabin view:

Mode   |   Trigger Condition   |   Normalized Crop Box $[y_{\min}, y_{\max}, x_{\min}, x_{\max}]$   |   Target Geometry

DASHCAM Mode (Default)   |   [pothole_test_2.mp4](file:///e:/Pothole%20detection/pothole_test_2.mp4) or bumper mount   |   y: [44%, 92%], x: [8%, 92%]   |   Focuses on the lower 50% of the visual field where the road surface exists. Filters out horizon, sky, trees, and oncoming traffic.

CABIN Mode   |   [test_video.mp4](file:///e:/Pothole%20detection/test_video.mp4) or --mode cabin   |   y: [22%, 49%], x: [28%, 67%]   |   Tightly bounds the windshield glass opening. Masks out the dashboard, hood reflections, rearview mirror, and right A-pillar.

---

### 6. Spatial Corridor & Proximity Avoidance Decision Engine

### 6.1 Road Corridor Partitioning

The road Region of Interest is divided horizontally into three equal longitudinal corridors:

0% ────────────────────────── 33.3% ──────────────────────── 66.7% ──────────────────────── 100%

┌───────────────────────────────┬───────────────────────────────┬───────────────────────────────┐

│         LEFT CORRIDOR         │        CENTER CORRIDOR        │        RIGHT CORRIDOR         │

│          [0.00, 0.33]         │          [0.33, 0.67]         │          [0.67, 1.00]         │

│                               │                               │                               │

│ If pothole detected here:     │ If pothole detected here:     │ If pothole detected here:     │

│   Risk in Left rises          │   IMMEDIATE COLLISION PATH    │   Risk in Right rises         │

│   >> DIRECTIVE: SWERVE RIGHT  │   >> EVADE TO CLEAR CORRIDOR  │   >> DIRECTIVE: SWERVE LEFT   │

└───────────────────────────────┴───────────────────────────────┴───────────────────────────────┘

### 6.2 Continuous Danger Scoring Formula

For each detection $i$ mapped to corridor $z \in \{\text{left}, \text{center}, \text{right}\}$:

$$\text{Danger}_i = \text{Conf}_i \times W_{\text{type}} \times \left(0.5 + 0.8 \cdot \frac{y_{\text{center}} - y_{\text{roi\_top}}}{h_{\text{roi}}}\right) \times \left(0.3 + 0.7 \cdot \frac{\text{Area}_i}{\text{RoadArea}}\right)$$

• Class Risk Multiplier ($W_{\text{type}}$):

* Pothole (D40): $W = 2.2$ (Immediate axle/suspension damage hazard)

* Alligator / Transverse Crack (D10, D20): $W = 1.5$ (Structural roughness)

* Longitudinal Crack (D00): $W = 1.2$

* Positive Obstacle (Car, Person): $W = 1.0$

• Vertical Proximity Weighting: Objects near the bottom of the ROI have higher $y_{\text{center}}$, scaling risk up to $1.3\times$ as the vehicle closes the physical distance.

• Corridor Aggregation: $\text{Score}_z = \min\left(1.0, \sum_{i \in z} \text{Danger}_i\right)$.

### 6.3 Reactive Decision Matrix

Threshold for corridor obstruction: $\tau = 0.20$.

Left Risk ($L$)   |   Center Risk ($C$)   |   Right Risk ($R$)   |   Primary State   |   HUD Directive Display   |   Serial Command Output

$< \tau$   |   $< \tau$   |   $< \tau$   |   CLEAR   |   PATH CLEAR -- CRUISING   |   DRIVE 180

$\ge \tau$   |   $< \tau$   |   $< \tau$   |   SWERVE_RIGHT   |   POTHOLE ON LEFT >> SWERVE RIGHT   |   TURN 30 150

$< \tau$   |   $< \tau$   |   $\ge \tau$   |   SWERVE_LEFT   |   << POTHOLE ON RIGHT: SWERVE LEFT   |   TURN -30 150

Low ($L < R$)   |   $\ge \tau$   |   High   |   EVADE_LEFT   |   << POTHOLE AHEAD: SWERVE LEFT   |   TURN -35 150

High   |   $\ge \tau$   |   Low ($R \le L$)   |   EVADE_RIGHT   |   POTHOLE AHEAD: SWERVE RIGHT >>   |   TURN 35 150

$\ge \tau$   |   $\ge \tau$   |   $\ge \tau$   |   STOP   |   !! HAZARD AHEAD: FULL STOP !!   |   PI_STOP

---

### 7. Real-Time HUD & Telemetry Visualizer

[run_detection.py](file:///e:/Pothole%20detection/run_detection.py) includes a high-definition telemetry rendering engine:

• Top Header Glassmorphism Bar: Displays active model checkpoint, device (CUDA/CPU), inference latency (ms), instantaneous FPS, and current camera perspective mode.

• Dynamic Corridor Danger Meters: Real-time percentage bars for Left, Center, and Right corridors, color-coded:

* Green: $0\% - 19\%$ (Safe)

* Yellow: $20\% - 49\%$ (Caution)

* Red: $\ge 50\%$ (Hazard)

• High-Visibility Action Banner: Large centered pill badge indicating the current avoidance directive (PATH CLEAR, SWERVE RIGHT, SWERVE LEFT, STOP / BRAKE).

• Road Corridor Overlay: Semi-transparent colored column overlays on the road plane showing safe vs. hazardous driving zones.

• Bounding Box Annotations: Color-coded bounding boxes showing class label, detection confidence, and spatial proximity indicator.

---

### 8. Repository Inventory & File Map

E:\Pothole detection\

│

├── CONTEXT.md                         # [THIS FILE] Master architecture, specs, and developer guide

├── run_detection.py                   # Production inference & HUD telemetry rendering script

├── train_pothole_model.py             # Automated training pipeline for YOLOv10n on local GPU

├── best.pt                            # Active 50-epoch fine-tuned weights (YOLOv10n, 5.75 MB)

├── yolov10n.pt                        # Base pretrained COCO weights

├── rdd2022_yolov8n.pt                 # Baseline benchmark weights

├── test_video.mp4                     # Source test video 1: In-cabin POV on damaged Indian road

├── pothole_test_2.mp4                 # Source test video 2: Pure front dashcam on wet highway

├── annotated_video.mp4                # Rendered verification video for test_video.mp4

├── annotated_pothole_test_2.mp4       # Rendered verification video for pothole_test_2.mp4

│

├── runs_rdd/custom_pothole_model/     # Training metrics, plots, and checkpoints

│   ├── weights/

│   │   ├── best.pt                    # Best checkpoint during training (identical to root best.pt)

│   │   └── last.pt                    # Final epoch checkpoint

│   ├── results.png                    # Loss and mAP convergence curves

│   ├── results.csv                    # Epoch-by-epoch loss & metric data

│   ├── confusion_matrix.png           # Class confusion matrix

│   ├── confusion_matrix_normalized.png# Normalized confusion matrix

│   ├── BoxF1_curve.png                # F1 confidence curve

│   ├── BoxPR_curve.png                # Precision-Recall curve

│   ├── val_batch0_labels.jpg          # Ground truth validation annotations

│   ├── val_batch0_pred.jpg            # Model inference on validation batch

│   └── args.yaml                      # Ultralytics training argument configuration

│

└── negative_obstacle_detection/

├── 05_pothole_ahead.md            # Hardware serial protocol & capstone milestone spec

├── README.md                      # Submodule overview

├── detector.py                    # Original edge wrapper module

├── prepare_rdd_yolo.py            # Pascal VOC XML to YOLO format converter

├── rdd_yolo_dataset/              # Clean YOLO-formatted dataset (train/val images & labels)

│   ├── rdd_dataset.yaml           # Dataset path configuration for Ultralytics

│   ├── train/ (1,224 images + label text files)

│   └── val/ (306 images + label text files)

└── train/                         # Raw Kaggle RDD2022 dataset (1,530 images + VOC XMLs)

> Cleanup Record: Outdated eval_metrics/ directory and redundant training mosaic dumps (train_batch*.jpg) have been permanently removed to keep the workspace clean and focused.

---

### 9. Operator & Developer CLI Guide

### 9.1 Inference & Video Processing

All commands run directly from PowerShell or Command Prompt:

## 1. Run detection on front dashcam video (Default mode: dashcam)

python run_detection.py --video pothole_test_2.mp4

## 2. Run with live interactive OpenCV playback window (Press Space to Pause, 'q' to Quit)

python run_detection.py --video pothole_test_2.mp4 --show

## 3. Run on in-cabin POV video (Automatically applies windshield mask)

python run_detection.py --video test_video.mp4 --show

## 4. Process only the first 30 seconds of a video

python run_detection.py --video pothole_test_2.mp4 --duration 30

## 5. Explicitly override perspective mode

python run_detection.py --video sample.mp4 --mode dashcam

python run_detection.py --video sample.mp4 --mode cabin

## 6. Specify custom weights or confidence threshold

python run_detection.py --video pothole_test_2.mp4 --weights best.pt --conf 0.35

### 9.2 Model Retraining / Fine-Tuning

## Retrain for 50 epochs on RTX 3050 GPU (~24 mins)

python train_pothole_model.py --epochs 50 --batch 8

## Fast 10-epoch sanity test

python train_pothole_model.py --epochs 10 --batch 8

---

### 10. Roadmap & Implementation Guide for Future Agents

Any agent or developer continuing this capstone project should execute the following three engineering tasks:

### Task 1: Temporal Exponential Moving Average (EMA) Smoothing

• Objective: Prevent single-frame detection drops from causing rapid 1-frame flickering between CLEAR and SWERVE.

• Implementation Plan in [run_detection.py](file:///e:/Pothole%20detection/run_detection.py):

Maintain a persistent dictionary of smoothed corridor risk scores across frames:

class CorridorSmoother:

def __init__(self, alpha: float = 0.70):

self.alpha = alpha  # Weight of current frame

self.smoothed = {"left": 0.0, "center": 0.0, "right": 0.0}

def update(self, raw_scores: dict) -> dict:

for k in self.smoothed:

self.smoothed[k] = (self.alpha * raw_scores.get(k, 0.0) +

(1.0 - self.alpha) * self.smoothed[k])

return self.smoothed

### Task 2: Hardware UART Serial Bridge Integration

• Objective: Send real-time motor control overrides from Python directly to the Arduino/ESP32 over USB serial.

• Implementation Plan:

import serial

import time

class VehicleSerialBridge:

def __init__(self, port: str = "COM3", baud: int = 115200):

self.ser = serial.Serial(port, baud, timeout=0.05)

time.sleep(2.0)  # Allow Arduino bootloader to settle

def send_command(self, cmd: str):

packet = f"{cmd.strip()}\n".encode('ascii')

self.ser.write(packet)

def check_telemetry(self):

if self.ser.in_waiting:

return self.ser.readline().decode('ascii', errors='ignore').strip()

return None

### Task 3: Inverse Perspective Mapping (IPM / Bird's-Eye View)

• Objective: Transform the 2D image perspective trapezoid into an orthographic top-down metric plane ($X, Y$ coordinates in centimeters).

Benefit: Allows the path planner to know exact physical distances (e.g., "Pothole is 120 cm forward, 25 cm right of vehicle centerline"*), enabling smooth Bezier curve spline evasion rather than fixed angular steering snaps.

# APPENDIX F — 01 CLEAR PATH

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 1: Clear Path — Normal Forward Driving

### Situation

The car is driving forward on an open path. No walls, no obstacles, no potholes. Pi is connected but has nothing to report.

### Initial State

STATE_IDLE — motors off, awaiting commands.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Pi → Arduino    DRIVE 180 5000

0ms      Arduino → Pi    [ACK] DRIVE 180 5000

0ms      Arduino → Pi    [TELEM] STATE=DRIVING SPEED=180

100ms    Arduino → Pi    [TELEM] US LEFT=85.2 RIGHT=92.1

200ms    Arduino → Pi    [TELEM] US LEFT=88.0 RIGHT=90.5

300ms    Arduino → Pi    [TELEM] US LEFT=84.7 RIGHT=91.8

...    (ultrasonic telemetry continues every 100ms)

5000ms   Arduino → Pi    [TELEM] DONE DRIVE

5000ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors

Forward drive   |   5000ms   |   PWM 180 forward   |   PWM 180 forward

Stop (timer expired)   |   —   |   0   |   0

### Final State

STATE_IDLE — motors off. Car stopped naturally after the timed duration.

### Key Takeaway

This is the simplest case. Ultrasonic readings are broadcast as telemetry but never trigger avoidance because all readings are well above 20cm.

# APPENDIX F — 02 WALL ON LEFT

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 2: Wall on the Left — Local Ultrasonic Avoidance

### Situation

The car is driving forward. A wall or object appears on the left side, within 20cm of the left HC-SR04P sensor. The Pi has not sent any commands — the Arduino handles this entirely on its own as a reflex.

### Initial State

STATE_DRIVING — moving forward at PWM 180.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Arduino → Pi    [TELEM] US LEFT=45.0 RIGHT=80.0     ← still clear

100ms    Arduino → Pi    [TELEM] US LEFT=28.3 RIGHT=78.5     ← getting closer

200ms    Arduino → Pi    [TELEM] US LEFT=15.0 RIGHT=78.0     ← ⚠️ LEFT < 20cm!

200ms    Arduino → Pi    [TELEM] AVOID RIGHT                  ← avoidance starts

--- Phase 1: Reverse (400ms) ---

(motors: left -150, right -150)

600ms    (internal)      Phase 1 complete → transition to Phase 2

--- Phase 2: Skid-Turn Right (500ms) ---

(motors: left +150, right -150)

1100ms   (internal)      Phase 2 complete → transition to Phase 3

1100ms   Arduino → Pi    [TELEM] DONE AVOID

--- Phase 3: Resume Forward (2000ms) ---

(motors: left +180, right +180 — resumes at saved speed)

3100ms   Arduino → Pi    [TELEM] DONE DRIVE

3100ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Why

Reverse   |   400ms   |   PWM 150 backward   |   PWM 150 backward   |   Back away from obstacle

Skid-turn right   |   500ms   |   PWM 150 forward   |   PWM 150 backward   |   Turn away from left wall

Resume forward   |   2000ms   |   PWM 180 forward   |   PWM 180 forward   |   Continue original path

### Final State

STATE_IDLE — car has cleared the obstacle and stopped after the resume timer.

### Key Takeaway

• Obstacle on left → car turns right (away from obstacle)

• The avoidance is fully autonomous — no Pi involvement needed

• The car resumes at the original drive speed (180), not the avoidance speed (150)

• If the Pi had previously sent PI_AVOID LEFT, the car would turn left instead (overriding the local decision)

# APPENDIX F — 03 WALL ON RIGHT

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 3: Wall on the Right — Local Ultrasonic Avoidance

### Situation

The car is driving forward. A wall or object appears on the right side, within 20cm of the right HC-SR04P sensor. Mirror image of Scenario 2.

### Initial State

STATE_DRIVING — moving forward at PWM 180.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Arduino → Pi    [TELEM] US LEFT=82.0 RIGHT=55.0     ← still clear

100ms    Arduino → Pi    [TELEM] US LEFT=80.5 RIGHT=30.2     ← getting closer

200ms    Arduino → Pi    [TELEM] US LEFT=79.0 RIGHT=12.4     ← ⚠️ RIGHT < 20cm!

200ms    Arduino → Pi    [TELEM] AVOID LEFT                   ← avoidance starts

--- Phase 1: Reverse (400ms) ---

(motors: left -150, right -150)

600ms    (internal)      Phase 1 complete

--- Phase 2: Skid-Turn Left (500ms) ---

(motors: left -150, right +150)

1100ms   Arduino → Pi    [TELEM] DONE AVOID

--- Phase 3: Resume Forward (2000ms) ---

(motors: left +180, right +180)

3100ms   Arduino → Pi    [TELEM] DONE DRIVE

3100ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Why

Reverse   |   400ms   |   PWM 150 backward   |   PWM 150 backward   |   Back away from obstacle

Skid-turn left   |   500ms   |   PWM 150 backward   |   PWM 150 forward   |   Turn away from right wall

Resume forward   |   2000ms   |   PWM 180 forward   |   PWM 180 forward   |   Continue original path

### Final State

STATE_IDLE — obstacle cleared.

### Key Takeaway

• Obstacle on right → car turns left (away from obstacle)

• Exact mirror of Scenario 2

# APPENDIX F — 04 BOTH SIDES BLOCKED

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 4: Both Sides Blocked — Narrow Passage

### Situation

The car is driving forward and enters a narrow gap (e.g., between two boxes, chair legs, or a doorframe). Both left AND right ultrasonics read below 20cm simultaneously.

### Initial State

STATE_DRIVING — moving forward at PWM 150.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Arduino → Pi    [TELEM] US LEFT=35.0 RIGHT=30.0     ← narrowing

100ms    Arduino → Pi    [TELEM] US LEFT=18.0 RIGHT=16.5     ← ⚠️ BOTH < 20cm!

100ms    Arduino → Pi    [TELEM] AVOID RIGHT                  ← default: turn right

--- Phase 1: Reverse (400ms) ---

(motors: left -150, right -150)

500ms    (internal)      Phase 1 complete

--- Phase 2: Skid-Turn Right (500ms) ---

(motors: left +150, right -150)

1000ms   Arduino → Pi    [TELEM] DONE AVOID

--- Phase 3: Resume Forward (2000ms) ---

(motors: left +150, right +150)

3000ms   Arduino → Pi    [TELEM] DONE DRIVE

3000ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Why

Reverse   |   400ms   |   PWM 150 backward   |   PWM 150 backward   |   Back out of narrow gap

Skid-turn right   |   500ms   |   PWM 150 forward   |   PWM 150 backward   |   Arbitrary default (right)

Resume forward   |   2000ms   |   PWM 150 forward   |   PWM 150 forward   |   Try a new path

### Final State

STATE_IDLE — car backed out and turned away.

### Key Takeaway

• When both sides are blocked, the Arduino defaults to turning right (arbitrary fallback)

• If the Pi has set PI_AVOID LEFT beforehand, the car will turn left instead

• This scenario may re-trigger avoidance if the car drives into another blocked area after resuming — that's expected (the car will keep trying to find a clear path)

• For a dead-end situation, the Pi's camera algorithm should detect it and take over with PI_STOP + manual TURN/DRIVE navigation

# APPENDIX F — 05 POTHOLE AHEAD

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 5: Pothole Ahead — Pi Camera Detection (No Physical Obstacle)

### Situation

The car is driving forward on a clear path. Ultrasonics read clear on both sides (no walls). The Pi 5's camera + YOLO algorithm detects a pothole (negative obstacle) directly ahead. The ultrasonic sensors cannot detect potholes — this is purely a camera/vision task.

### Initial State

STATE_DRIVING — moving forward at PWM 180.

### Why ultrasonics can't help here

HC-SR04P sensors fire sound pulses horizontally to the left and right. A pothole is a hole in the ground — there's nothing for the sound to bounce off of. Only the camera looking forward and down can see it.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Arduino → Pi    [TELEM] US LEFT=90.0 RIGHT=88.0     ← all clear

0ms      Arduino → Pi    [TELEM] STATE=DRIVING SPEED=180

(Pi's YOLO algorithm detects pothole in camera frame)

150ms    Pi → Arduino    PI_STOP

150ms    Arduino → Pi    [ACK] PI_STOP

150ms    Arduino → Pi    [TELEM] STATE=PI_STOPPED

(Car is now completely frozen. Ultrasonics still polled

but avoidance CANNOT trigger — Pi latch overrides.)

(Pi's algorithm analyzes the scene, decides left is clear)

300ms    Pi → Arduino    PI_AVOID LEFT                         ← hint stored

300ms    Arduino → Pi    [ACK] PI_AVOID LEFT

(Pi releases the latch)

500ms    Pi → Arduino    PI_GO

500ms    Arduino → Pi    [ACK] PI_GO

500ms    Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

(Pi now navigates the car around the pothole manually)

600ms    Pi → Arduino    TURN -60 150                          ← turn left 60°

600ms    Arduino → Pi    [ACK] TURN -60 150

600ms    Arduino → Pi    [TELEM] STATE=TURNING

1200ms   Arduino → Pi    [TELEM] DONE TURN

1200ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

1300ms   Pi → Arduino    DRIVE 150 2000                        ← drive past pothole

1300ms   Arduino → Pi    [ACK] DRIVE 150 2000

1300ms   Arduino → Pi    [TELEM] STATE=DRIVING SPEED=150

3300ms   Arduino → Pi    [TELEM] DONE DRIVE

3400ms   Pi → Arduino    TURN 60 150                           ← straighten out

3400ms   Arduino → Pi    [ACK] TURN 60 150

4000ms   Arduino → Pi    [TELEM] DONE TURN

4100ms   Pi → Arduino    DRIVE 180 5000                        ← resume normal speed

4100ms   Arduino → Pi    [ACK] DRIVE 180 5000

### Motor Behavior

Phase   |   Who decides   |   Duration   |   Left Motors   |   Right Motors

Driving forward   |   Pi (DRIVE cmd)   |   until PI_STOP   |   PWM 180 fwd   |   PWM 180 fwd

FROZEN   |   Pi (PI_STOP)   |   ~350ms   |   0   |   0

Turn left 60°   |   Pi (TURN cmd)   |   ~600ms   |   PWM 150 backward   |   PWM 150 forward

Drive past pothole   |   Pi (DRIVE cmd)   |   2000ms   |   PWM 150 forward   |   PWM 150 forward

Turn right 60°   |   Pi (TURN cmd)   |   ~600ms   |   PWM 150 forward   |   PWM 150 backward

Resume driving   |   Pi (DRIVE cmd)   |   5000ms   |   PWM 180 forward   |   PWM 180 forward

### Final State

STATE_DRIVING — car has navigated around the pothole and is back on course.

### Key Takeaway

• Ultrasonics are blind to potholes. Only the camera can see them.

• The Pi is the brain here — it stops the car, plans the route, and sends step-by-step commands.

• The Arduino just executes TURN and DRIVE commands as instructed.

• If a wall suddenly appears during the Pi's maneuver, the ultrasonic reflex (Tier 2) will interrupt and avoid it — then the Pi can re-plan.

• The PI_AVOID LEFT hint sent at 300ms would only be used if an ultrasonic trigger happened — since it didn't, the hint stays stored (cleared on next use or overwritten by a new PI_AVOID).

# APPENDIX F — 06 PI STOP DURING AVOIDANCE

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 6: Pi Interrupts Local Avoidance Mid-Maneuver

### Situation

The car's left ultrasonic detected a wall and started a local avoidance maneuver (reversing). While the car is mid-reverse, the Pi's camera detects a person stepping into the path behind the car. The Pi sends PI_STOP immediately. Tier 1 overrides Tier 2 mid-maneuver.

### Initial State

STATE_AVOIDING, AVOID_REVERSE phase — car is currently reversing as part of a Tier 2 avoidance.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Arduino → Pi    [TELEM] US LEFT=12.0 RIGHT=75.0     ← obstacle left

0ms      Arduino → Pi    [TELEM] AVOID RIGHT                  ← avoidance started

--- Phase 1: Reverse (started, expected to last 400ms) ---

(motors: left -150, right -150)

(Pi camera sees person behind the car at ~200ms)

200ms    Pi → Arduino    PI_STOP                               ← FREEZE!

200ms    Arduino → Pi    [ACK] PI_STOP

200ms    Arduino → Pi    [TELEM] STATE=PI_STOPPED

--- Avoidance KILLED mid-phase. Motors = 0. ---

--- Car is frozen. The remaining 200ms of reverse = cancelled. ---

--- The skid-turn phase = cancelled. ---

--- The resume phase = cancelled. ---

(Person walks away)

3000ms   Pi → Arduino    PI_GO

3000ms   Arduino → Pi    [ACK] PI_GO

3000ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

(Clean slate. Avoidance is NOT resumed.)

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Note

Avoidance reverse   |   200ms (of 400ms)   |   PWM 150 backward   |   PWM 150 backward   |   Interrupted

FROZEN   |   ~2800ms   |   0   |   0   |   Pi latch active

Idle   |   —   |   0   |   0   |   Clean slate

### Final State

STATE_IDLE — the avoidance maneuver was not resumed. The car starts fresh. The Pi (or ultrasonics on next poll) will decide what happens next.

### Key Takeaway

• PI_STOP kills everything instantly — even a Tier 2 avoidance in progress

• The avoidance is not resumed after PI_GO. The car goes to a clean STATE_IDLE.

• This is a safety feature: if the Pi sees danger, the car must stop immediately regardless of what the Arduino's local logic is doing

• After PI_GO, the ultrasonic sensors will re-poll on the next 100ms cycle and may trigger a new avoidance if the original wall is still there

# APPENDIX F — 07 PI DIRECTED AVOIDANCE

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 7: Pi-Directed Avoidance via PI_AVOID Hint

### Situation

The car is driving forward. The Pi's camera sees that the right side has a drop-off (cliff edge) that the ultrasonics can't detect. The Pi sends PI_AVOID LEFT as a preventive hint. Later, the left ultrasonic detects a wall. Normally the car would turn right (away from the left wall), but the Pi's hint overrides: the car turns left instead, because the Pi knows the right side is dangerous.

### Initial State

STATE_DRIVING — moving forward at PWM 180.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

(Pi camera sees cliff edge on the right side)

0ms      Pi → Arduino    PI_AVOID LEFT                        ← hint stored

0ms      Arduino → Pi    [ACK] PI_AVOID LEFT

(Car continues driving normally)

100ms    Arduino → Pi    [TELEM] US LEFT=60.0 RIGHT=85.0     ← clear

200ms    Arduino → Pi    [TELEM] US LEFT=40.0 RIGHT=80.0     ← clear

300ms    Arduino → Pi    [TELEM] US LEFT=15.0 RIGHT=78.0     ← ⚠️ LEFT < 20cm!

(Normally: obstacle left → turn RIGHT)

(But PI_AVOID LEFT is stored → override to turn LEFT)

300ms    Arduino → Pi    [TELEM] AVOID LEFT                   ← Pi hint used!

--- Phase 1: Reverse (400ms) ---

(motors: left -150, right -150)

700ms    (internal)      Phase 1 complete

--- Phase 2: Skid-Turn LEFT (500ms) ---

(motors: left -150, right +150)

(This is OPPOSITE of what the Arduino would have done locally!)

1200ms   Arduino → Pi    [TELEM] DONE AVOID

--- Phase 3: Resume Forward (2000ms) ---

(motors: left +180, right +180)

3200ms   Arduino → Pi    [TELEM] DONE DRIVE

3200ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Note

Reverse   |   400ms   |   PWM 150 backward   |   PWM 150 backward   |   Back away

Skid-turn LEFT   |   500ms   |   PWM 150 backward   |   PWM 150 forward   |   Pi hint override!

Resume forward   |   2000ms   |   PWM 180 forward   |   PWM 180 forward   |   Continue

### Without the Pi hint (default behavior)

The car would have turned RIGHT (away from the left wall) — directly toward the cliff edge the Pi detected. The PI_AVOID LEFT hint prevented this.

### Final State

STATE_IDLE — car avoided the wall while staying away from the cliff edge.

### Key Takeaway

• PI_AVOID is a one-shot hint — it's consumed (cleared) after one use

• It overrides the Arduino's local decision about which direction to turn

• The Pi uses this when it has information the ultrasonics don't (cliff edges, drop-offs, specific obstacles visible only to the camera)

• If no ultrasonic trigger happens, the hint just sits there until the next trigger or until overwritten by a new PI_AVOID

# APPENDIX F — 08 BENCH TEST NO PI

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 8: Bench Test — No Pi Connected (Serial Monitor Only)

### Situation

You're testing with just the Arduino Uno and a USB cable to your laptop. No Pi is connected. You're using the Arduino IDE Serial Monitor to send commands and observe telemetry. This is how you'll test when the Uno is available but the Pi isn't set up yet.

### Initial State

Power on → Arduino runs setup() → STATE_IDLE.

### Serial Monitor Setup

• Baud rate: 9600

• Line ending: Newline (\n)

### Serial Message Flow

TIME     DIRECTION          MESSAGE

─────────────────────────────────────────────────────

Arduino → Monitor  [TELEM] READY Uno-ATmega328P

── Test 1: Basic driving ──

You type: DRIVE 150 3000

Arduino → Monitor  [ACK] DRIVE 150 3000

Arduino → Monitor  [TELEM] STATE=DRIVING SPEED=150

Arduino → Monitor  [TELEM] US LEFT=100.0 RIGHT=95.0

Arduino → Monitor  [TELEM] US LEFT=98.5 RIGHT=97.2

...

3s later Arduino → Monitor  [TELEM] DONE DRIVE

Arduino → Monitor  [TELEM] STATE=IDLE SPEED=0

── Test 2: Manual stop ──

You type: DRIVE 200 10000

Arduino → Monitor  [ACK] DRIVE 200 10000

(car moves)

You type: STOP

Arduino → Monitor  [ACK] STOP

Arduino → Monitor  [TELEM] STATE=IDLE SPEED=0

── Test 3: Ultrasonic avoidance ──

You type: DRIVE 150 10000

(car moves)

(you place your hand ~10cm from the left sensor)

Arduino → Monitor  [TELEM] US LEFT=10.2 RIGHT=88.0

Arduino → Monitor  [TELEM] AVOID RIGHT

(car reverses, turns right, resumes)

Arduino → Monitor  [TELEM] DONE AVOID

── Test 4: Simulate Pi commands ──

You type: PI_STOP

Arduino → Monitor  [ACK] PI_STOP

Arduino → Monitor  [TELEM] STATE=PI_STOPPED

(car frozen — try sending DRIVE, it won't work)

You type: DRIVE 150 2000

Arduino → Monitor  [ACK] BLOCKED (PI_STOPPED)

You type: PI_GO

Arduino → Monitor  [ACK] PI_GO

Arduino → Monitor  [TELEM] STATE=IDLE SPEED=0

(car unfrozen, DRIVE works again)

── Test 5: Pi avoid hint ──

You type: PI_AVOID LEFT

Arduino → Monitor  [ACK] PI_AVOID LEFT

You type: DRIVE 150 10000

(place hand near RIGHT sensor)

Arduino → Monitor  [TELEM] US LEFT=85.0 RIGHT=8.0

Arduino → Monitor  [TELEM] AVOID LEFT

(car turns LEFT even though obstacle is on RIGHT — hint override!)

── Test 6: Reverse and turn ──

You type: REV 150 2000

Arduino → Monitor  [ACK] REV 150 2000

(car reverses)

You type: TURN 90 150

Arduino → Monitor  [ACK] TURN 90 150

(car skid-turns right for ~900ms)

── Test 7: Ping ──

You type: PING

Arduino → Monitor  [ACK] PONG

### Final State

Depends on last command sent. The car is fully controllable from Serial Monitor.

### Key Takeaway

• Everything works without the Pi — Serial Monitor acts as a substitute

• You can simulate PI_STOP, PI_GO, PI_AVOID from Serial Monitor to test the full priority system

• This is useful for:

- Calibrating avoidance timing on the real chassis

- Testing ultrasonic sensor placement and readings

- Verifying motor direction and speed

- Demonstrating the system to teammates

# APPENDIX F — 09 OBSTACLE WHILE REVERSING

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 9: Obstacle Detected While Reversing

### Situation

The Pi told the car to reverse (e.g., backing away from a pothole). While reversing, the right ultrasonic detects an obstacle. The Arduino's reflex triggers avoidance even during a reverse command.

### Initial State

STATE_REVERSING — moving backward at PWM 150 (from a REV 150 2000 command).

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

0ms      Pi → Arduino    REV 150 2000

0ms      Arduino → Pi    [ACK] REV 150 2000

0ms      Arduino → Pi    [TELEM] STATE=REVERSING SPEED=150

100ms    Arduino → Pi    [TELEM] US LEFT=80.0 RIGHT=45.0     ← clear

200ms    Arduino → Pi    [TELEM] US LEFT=78.0 RIGHT=18.5     ← ⚠️ RIGHT < 20cm!

200ms    Arduino → Pi    [TELEM] AVOID LEFT                   ← avoidance triggered

--- Phase 1: Reverse (400ms) ---

(Note: car was ALREADY reversing, now it reverses at avoidance speed)

(motors: left -150, right -150)

600ms    (internal)      Phase 1 complete

--- Phase 2: Skid-Turn Left (500ms) ---

(motors: left -150, right +150)

1100ms   Arduino → Pi    [TELEM] DONE AVOID

--- Phase 3: Resume Forward (2000ms) ---

(motors: left +150, right +150)

(Note: savedDriveSpeed was 0 during REV, so uses AVOID_SPEED)

3100ms   Arduino → Pi    [TELEM] DONE DRIVE

3100ms   Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

### Motor Behavior

Phase   |   Duration   |   Left Motors   |   Right Motors   |   Note

Original reverse   |   200ms (of 2000ms)   |   PWM 150 backward   |   PWM 150 backward   |   Interrupted by ultrasonic

Avoidance reverse   |   400ms   |   PWM 150 backward   |   PWM 150 backward   |   Additional reverse

Skid-turn left   |   500ms   |   PWM 150 backward   |   PWM 150 forward   |   Away from right obstacle

Resume forward   |   2000ms   |   PWM 150 forward   |   PWM 150 forward   |   At AVOID_SPEED (no saved drive speed from REV)

### Final State

STATE_IDLE — car has cleared the obstacle.

### Key Takeaway

• Ultrasonic avoidance triggers during any movement state: DRIVING, REVERSING, or TURNING

• When the original command was REV, savedDriveSpeed is 0, so the resume phase uses AVOID_SPEED (150) instead

• The original REV command's remaining duration is lost — the car doesn't go back to reversing after avoidance

# APPENDIX F — 10 RAPID PI COMMANDS

Test-specification status: expected behavior only. Times, distances, and telemetry samples in this file are illustrative unless a dated raw run log is attached.

## Scenario 10: Rapid Pi Commands — Queuing Behavior

### Situation

The Pi sends multiple commands in quick succession. This tests how the Arduino handles command overlap — important because the Pi's algorithm may make rapid decisions.

### Initial State

STATE_IDLE — motors off.

### Key Behavior

The Arduino processes commands sequentially via Serial.readStringUntil('\n'). Each new driving command replaces the previous one (no queue). PI_STOP always takes effect immediately.

### Serial Message Flow

TIME     DIRECTION       MESSAGE

─────────────────────────────────────────────────────

── Case A: New DRIVE replaces old DRIVE ──

0ms      Pi → Arduino    DRIVE 180 5000

0ms      Arduino → Pi    [ACK] DRIVE 180 5000

(car starts at PWM 180)

100ms    Pi → Arduino    DRIVE 120 3000                       ← new command

100ms    Arduino → Pi    [ACK] DRIVE 120 3000

(car immediately switches to PWM 120, timer resets to 3000ms)

(the original 5000ms command is gone)

3100ms   Arduino → Pi    [TELEM] DONE DRIVE

── Case B: TURN interrupts DRIVE ──

0ms      Pi → Arduino    DRIVE 180 5000

100ms    Pi → Arduino    TURN 45 150                          ← interrupts

100ms    Arduino → Pi    [ACK] TURN 45 150

(car immediately starts turning, drive is abandoned)

550ms    Arduino → Pi    [TELEM] DONE TURN

── Case C: PI_STOP during rapid commands ──

0ms      Pi → Arduino    DRIVE 180 5000

50ms     Pi → Arduino    PI_STOP                              ← overrides immediately

50ms     Arduino → Pi    [ACK] PI_STOP

50ms     Arduino → Pi    [TELEM] STATE=PI_STOPPED

(car frozen after only 50ms of movement)

100ms    Pi → Arduino    DRIVE 150 3000                       ← sent while stopped

100ms    Arduino → Pi    [ACK] BLOCKED (PI_STOPPED)           ← rejected!

200ms    Pi → Arduino    PI_GO

200ms    Arduino → Pi    [ACK] PI_GO

200ms    Arduino → Pi    [TELEM] STATE=IDLE SPEED=0

300ms    Pi → Arduino    DRIVE 150 3000                       ← NOW it works

300ms    Arduino → Pi    [ACK] DRIVE 150 3000

── Case D: Multiple PI_AVOID (last one wins) ──

0ms      Pi → Arduino    PI_AVOID LEFT

0ms      Arduino → Pi    [ACK] PI_AVOID LEFT

100ms    Pi → Arduino    PI_AVOID RIGHT                       ← overwrites LEFT

100ms    Arduino → Pi    [ACK] PI_AVOID RIGHT

(if ultrasonic triggers now, car turns RIGHT — last hint wins)

### Key Takeaway

• No command queue — each new driving command replaces the previous one immediately

• PI_STOP is instant — cuts through anything, even commands sent in the same serial buffer

• Commands sent during PI_STOPPED are rejected with a BLOCKED acknowledgment

• PI_AVOID is overwritable — the last hint sent before an ultrasonic trigger is the one that gets used

• The Pi should wait for [ACK] responses before sending the next command if ordering matters (e.g., the pothole navigation sequence in Scenario 5)

# APPENDIX G — MODEL TRAINING RECORD AND REPRODUCIBILITY

The training results CSV contains 50 epoch records. The final row reports box loss 1.71172, classification loss 1.91510, precision 0.53992, recall 0.27092, mAP@0.5 0.25226, and mAP@0.5:0.95 0.11421. At epoch 1, box loss was 2.01838 and classification loss was 6.06841. These are values transcribed from the local CSV, not newly generated measurements. Confirm the data split and training arguments before describing them as held-out validation performance.

The training script defaults to 25 epochs, batch size 8, image size 640, and two workers, but the CSV has 50 epochs. The script searches for an RDD YOLO dataset YAML that is not present in the scanned repository inventory. It loads a YOLOv10n base checkpoint if locally available. Adjacent source notes refer to YOLOv8n in places, so model family and weight identity must be reconciled. For a reproducible report, include the dataset version and subset, class map, split manifest, exact command, software versions, base checkpoint hash, selected weights hash, and random seeds.

The CSV curves show training losses and validation metrics per epoch, but a single final metric row does not show the full error distribution. Use the included precision/recall and confusion plots only after confirming they were generated from the same checkpoint and data split. Include example false positives and missed potholes, particularly shadows, cracks, patched surfaces, small distant cavities, and partially cropped detections. Do not infer field readiness from a benchmark dataset metric alone.

# APPENDIX H — PI 5 BENCHMARKING PROCEDURE AND CLAIM AUDIT

The benchmark source configures thread counts, limits OpenCV to one thread, preloads video frames to reduce file I/O effects, warms the model before timing, disables garbage collection during the timed loop, and computes average FPS, mean/median/P95/P99 latency, latency standard deviation, memory, temperature, and throttle status. It can compare multiple core allocations and model engines. Source defaults include 100 preloaded frames and 25 warm-up iterations. Record the actual values passed to each run rather than assuming defaults.

No dated benchmark result was present under benchmarking/results; the directory contained only .gitkeep. Therefore the repository’s claims of 22–26 ms per YOLOv10n NCNN frame, 38–45 FPS, less than 4 ms classical pothole processing, approximately 1.2 ms serial round trip, less than 10 ms local reflex, less than 4 microseconds PWM jitter, less than 1.5 cm stopping distance, and 52 °C steady temperature are not supported by a visible raw benchmark result in that directory. The final report should either attach source logs and methodology or retain these only as unverified project-document claims.

A complete benchmark log should contain date and time, host board and memory configuration, OS release, runtime/library versions, CPU governor status, cooling, ambient temperature, model filename and hash, input dimensions, number of frames, warm-up count, core/thread configuration, per-frame timings, start/end temperature, throttle bitmask, and output CSV/JSON. Run at least three independent sessions and preserve all runs. Report central tendency and tail latency, not only the single fastest observation.

# APPENDIX I — CONSOLIDATED NUMERICAL SETTINGS AND CONFLICTS

# REFERENCES

[1] J. Redmon, S. Divvala, R. Girshick, and A. Farhadi, “You Only Look Once: Unified, Real-Time Object Detection,” in Proc. IEEE Conf. Computer Vision and Pattern Recognition (CVPR), 2016, pp. 779–788, doi: 10.1109/CVPR.2016.91.

[2] A. Wang, H. Chen, L. Liu, K. Chen, Z. Lin, J. Han, and G. Ding, “YOLOv10: Real-Time End-to-End Object Detection,” arXiv:2405.14458, 2024.

[3] D. Arya, H. Maeda, S. K. Ghosh, D. Toshniwal, and Y. Sekimoto, “RDD2022: A multi-national image dataset for automatic road damage detection,” Geoscience Data Journal, doi: 10.1002/gdj3.260, 2024.

[4] D. Arya et al., “Crowdsensing-based Road Damage Detection Challenge (CRDDC’2022),” in Proc. IEEE Int. Conf. Big Data, 2022, pp. 6378–6386, doi: 10.1109/BigData55660.2022.10021040.

[5] A. Geiger, P. Lenz, C. Stiller, and R. Urtasun, “Vision meets robotics: The KITTI dataset,” Int. J. Robotics Research, vol. 32, no. 11, pp. 1231–1237, 2013, doi: 10.1177/0278364913491297.

[6] J. F. Henriques, R. Caseiro, P. Martins, and J. Batista, “High-speed tracking with kernelized correlation filters,” IEEE Trans. Pattern Analysis and Machine Intelligence, vol. 37, no. 3, pp. 583–596, Mar. 2015, doi: 10.1109/TPAMI.2014.2345390.

[7] M. Sandler, A. Howard, M. Zhu, A. Zhmoginov, and L.-C. Chen, “MobileNetV2: Inverted residuals and linear bottlenecks,” in Proc. IEEE/CVF Conf. Computer Vision and Pattern Recognition (CVPR), 2018, pp. 4510–4520.

[8] M. Müller and V. Koltun, “OpenBot: Turning smartphones into robots,” in Proc. IEEE Int. Conf. Robotics and Automation (ICRA), 2021, doi: 10.1109/ICRA48506.2021.9561788.

[9] L. Paull et al., “Duckietown: An open, inexpensive and flexible platform for autonomy education and research,” in Proc. IEEE Int. Conf. Robotics and Automation (ICRA), 2017, doi: 10.1109/ICRA.2017.7989179.

[10] M. Quigley et al., “ROS: An open-source Robot Operating System,” in Proc. ICRA Workshop on Open Source Software, 2009.

[11] D. Fox, W. Burgard, and S. Thrun, “The dynamic window approach to collision avoidance,” IEEE Robotics & Automation Magazine, vol. 4, no. 1, pp. 23–33, Mar. 1997, doi: 10.1109/100.580977.

[12] J. Canny, “A computational approach to edge detection,” IEEE Trans. Pattern Analysis and Machine Intelligence, vol. PAMI-8, no. 6, pp. 679–698, Nov. 1986, doi: 10.1109/TPAMI.1986.4767851.

[13] N. Otsu, “A threshold selection method from gray-level histograms,” IEEE Trans. Systems, Man, and Cybernetics, vol. 9, no. 1, pp. 62–66, Jan. 1979, doi: 10.1109/TSMC.1979.4310076.

[14] Tencent, “NCNN: High-performance neural network inference framework optimized for mobile platforms,” GitHub repository. [Online]. Available: https://github.com/Tencent/ncnn.

[15] STMicroelectronics, “L298: Dual full-bridge driver,” datasheet DS0218, Rev. 5. [Online]. Available: https://www.st.com/resource/en/datasheet/cd00000240.pdf.

[16] Microchip Technology, “ATmega328P 8-bit AVR microcontroller with 32K bytes in-system programmable flash,” datasheet.

[17] Raspberry Pi Ltd., “Raspberry Pi 5 and BCM2712 documentation.” [Online]. Available: https://www.raspberrypi.com/documentation/computers/processors.html.

[18] OpenCV, “Tracking API: TrackerKCF,” OpenCV documentation. [Online]. Available: https://docs.opencv.org/.

[19] K. J. Åström and R. M. Murray, Feedback Systems: An Introduction for Scientists and Engineers. Princeton, NJ, USA: Princeton Univ. Press, 2008.

[20] Project repository, “System architecture, serial protocol, source code, training outputs, and driving scenario specifications,” model-car-capstone, accessed Oct. 24, 2026.