DESIGN, IMPLEMENTATION, AND COMPARATIVE BENCHMARKING OF AN AUTONOMOUS FOLLOW-ME AND OBSTACLE-AVOIDANT ROBOTIC VEHICLE

A Two-Layer Heterogeneous Computing Architecture (Raspberry Pi 5 + Arduino Uno) with NMS-Free YOLOv10n Inference, RDD2022 Pothole Avoidance, and Multi-Tier Reflex Control

| Academic Track: B.Tech Final Year Capstone Project | Target Hardware: Raspberry Pi 5 (8GB) + Arduino Uno (ATmega328P) |
| --- | --- |
| Project Repositories: model-car-capstone | Evaluation Document: Interim Technical Report (Aligned with Table 5.2.5 Rubrics) |
| Author Team: Aditya Rana, Parth Surve, Krishna Menon | Review Milestone: 2nd Review Evaluation |

1. Executive Summary & Problem Formulation [Rubric B: Problem Statement & Research Gaps]

――――――――――――――――――――――――――――――――――――――――――――――――――――

Autonomous mobile robotics in developing road environments (particularly Indian urban and semi-urban conditions) faces a harsh operational reality: high-throughput computer vision must operate under severe computational constraints, while simultaneously guaranteeing microsecond-accurate motor actuation and reflexive physical safety. Most educational and open-source autonomous robotics platforms (such as NVIDIA JetBot, DonkeyCar, and JetRacer) collapse both perception and motor actuation onto a single Linux compute board. In practice, this monolithic design fails when scaled to real-world edge autonomy.

1.1 Identified Research Gaps

Research Gap 1 — The Linux OS Scheduling & PWM Jitter Dilemma: General-purpose Linux operating systems rely on the Completely Fair Scheduler (CFS), which provides no deterministic timing guarantees. When executing heavy deep neural networks, interrupt latency spikes from <50 µs to over 20–50 ms. Generating motor PWM via software GPIO results in severe duty-cycle distortion, motor shudder, and erratic skid-steer drift. When a sudden obstacle appears, the 60–120 ms round-trip perception latency means the vehicle travels 10–18 cm before braking begins, causing physical collision.

Research Gap 2 — Acoustic Blindness to Negative Obstacles (Potholes): Ultrasonic distance sensors (such as the ubiquitous HC-SR04P) propagate acoustic waves horizontally. When encountering negative obstacles (potholes, downward road fractures, uncovered drainage cavities, or steep drop-offs), the sound wave travels over the depression with zero backscatter, returning a false 'path clear' (infinite distance) reading. On Indian roads, where potholes cause thousands of vehicular accidents annually, relying on ultrasound alone is catastrophic.

Research Gap 3 — The Indian Hardware Procurement & Cost Barrier: Commercial edge AI robotics kits (JetBot, JetRacer, AWS DeepRacer) cost $250–$400 USD. In India, due to import tariffs, customs duties, and distributor margins, these platforms cost ₹25,000 to ₹35,000+ INR. There is an acute research and educational gap for an autonomous vehicle architecture that achieves superior multi-modal reflex safety at a fraction of this cost using locally accessible, modular hardware.

1.2 Stated Project Objectives

Core Objectives: To resolve these research gaps, this capstone project achieves four concrete engineering objectives:

Objective 1: Design and validate a physically decoupled Two-Layer Heterogeneous Computing Architecture (Raspberry Pi 5 high-level perception + Arduino Uno ATmega328P hard real-time reflex spine) guaranteeing sub-microsecond hardware PWM timing.

Objective 2: Implement an NMS-free YOLOv10n neural detector via Tencent NCNN achieving sustained 30+ FPS (measured 22–26 ms latency) on Pi 5 CPU without requiring external hardware accelerators.

Objective 3: Formulate and train a dual-modality perception engine combining a fine-tuned RDD2022 YOLOv8n deep model with an ultra-lightweight (<4 ms) classical Laplacian CV fallback to detect negative obstacles (potholes) invisible to sonar.

Objective 4: Implement a 3-Tier Priority Finite State Machine in firmware providing autonomous sub-10 ms local ultrasonic reflex avoidance (Tier 2) and latched emergency overrides (Tier 1 PI_STOP) to ensure zero collisions during follow-me person tracking.

2. Literature Review & Comprehensive Market Survey [Rubric A: Understanding of Existing Research]

――――――――――――――――――――――――――――――――――――――――――――――――――――

To establish academic rigor and validate our architectural choices, we benchmarked our system against five prominent small-scale autonomous robotics platforms: NVIDIA JetBot, DonkeyCar, Waveshare/NVIDIA JetRacer, Intel/Stanford OpenBot, and standard SunFounder PiCar-X academic builds. The comparative metrics are summarized in Table 1.

| Platform | Compute Tier | India Retail Cost | Control Architecture | Vision Pipeline | Sensor Suite | Key Limitations & Trade-offs |
| --- | --- | --- | --- | --- | --- | --- |
| Our Vehicle (MentorPi / Capstone) | Raspberry Pi 5 (8GB Quad A76) + Arduino Uno (ATmega328P) | ₹23,900 Asset (₹4,929 out-of-pocket via college sourcing) | 2-Layer Heterogeneous Split (Pi Brain + Arduino Reflex Spine) | YOLOv10n (NCNN NMS-free) + Custom RDD2022 Pothole YOLO/CV | CSI Global-Shutter Cam + 2x HC-SR04P Ultrasonic + 2x IR Proximity | Dual-mode: Follow-me tracking & autonomous pothole avoidance. Hardware timer PWM, zero jitter, sub-10ms reflex. |
| NVIDIA JetBot | NVIDIA Jetson Nano (128-core Maxwell GPU, Quad A57) | ₹26,000–₹32,000 (Imported kit excl. battery) | Single-Board Monolithic (Nano handles vision & I2C motor driver) | SSD-MobileNet v2 / ResNet-18 (TensorRT optimized) | 8MP Sony IMX219 CSI Camera Only | Educational AI vehicle. High power draw; thermal throttling; Linux PWM timing jitter; zero distance sensors by default. |
| DonkeyCar | Raspberry Pi 3B+/4 + PCA9685 PWM Servo Shield | ₹24,000–₹28,000 (Standard RC chassis kit) | Single-Board Architecture with I2C servo expansion | Behavioral Cloning CNN (Keras / TensorFlow) | Wide-angle Pi Camera (No range sensors by default) | Closed-track lane/circuit racing. Lacks general object detection, person re-ID, and negative obstacle avoidance. |
| NVIDIA JetRacer | NVIDIA Jetson Nano + PCA9685 / Custom Carrier | ₹30,000–₹35,000 (Ackermann chassis) | Single-Board Monolithic (Jetson directly commands servo/ESC) | PyTorch → TensorRT (torch2trt steering model) | CSI Camera (IMX219) (No sonar / lidar) | High-speed racing focus; high inference FPS but zero reflex safety fallback, no sonar, and no follow-me tracking. |
| Intel / Stanford OpenBot | Consumer Smartphone (Android/iOS) + Arduino Nano body | ₹4,500 chassis body (Excludes ₹35,000+ smartphone) | 2-Layer Smartphone-as-Brain + Arduino motor slave | MobileNet SSD / YOLO-Lite (TFLite / CoreML on mobile NPU) | Phone Camera, IMU, GPS, Microphone + 1x Sonar on body | Pioneered person-following benchmark. Fragile phone mounting; severe thermal throttling after 5–10 min; non-embedded. |
| SunFounder PiCar-X / Standard Academic Builds | Raspberry Pi 4 / 3B + Robot HAT / Arduino Shield | ₹14,000–₹17,000 (Off-the-shelf kit) | Single/Co-processor board via simple serial loop | OpenCV Haar Cascades or MobileNet SSD v1 (CPU inference) | USB/CSI Camera + 1x Ultrasonic Sensor | Basic STEM educational vehicle. Low frame rates (3–8 FPS); obsolete ML models; primitive stop-and-turn obstacle routines. |

2.1 Detailed Synthesis of Literature & Architectural Debates

OpenBot (Müller & Koltun, IROS 2020) serves as the primary academic precedent for our follow-me use case. OpenBot demonstrated that decoupling high-level computer vision from a low-level microcontroller enables robust person following. However, OpenBot's reliance on a consumer smartphone introduces catastrophic risks for industrial and unstructured outdoor environments: phones suffer from battery swelling, lack active cooling (thermal throttling within 8 minutes), have fixed camera angles, and expose an expensive ₹40,000+ consumer device to chassis rollover damage. Our system preserves the architectural wisdom of OpenBot's two-layer partition while packaging it entirely within an embedded, ruggedized, actively cooled Single Board Computer (Raspberry Pi 5) paired with an ATmega328P microcontroller.

Furthermore, modern edge AI research (Wang et al., 2024; YOLOv10) has proven that end-to-end NMS-free models eliminate the Non-Maximum Suppression bottleneck that previously throttled embedded processors. By compiling YOLOv10n into Tencent NCNN with ARM NEON FP16 SIMD instructions, we achieve 30+ FPS on a standard quad-core Cortex-A76 CPU—rendering expensive, power-hungry discrete GPUs (like the Jetson Nano Maxwell) unnecessary.

3. System Design & Architectural Representation [Rubric C: Architecture & Algorithms]

――――――――――――――――――――――――――――――――――――――――――――――――――――

The system architecture is structured around complete physical and functional isolation. Figure 1 illustrates the high-level system topology, interconnects, and data flows.

📌 SYSTEM TOPOLOGY BLOCK DIAGRAM: SYSTEM TOPOLOGY & POWER BUS ARCHITECTURE: +-----------------------------------+ +----------------------------------+ ¦ Raspberry Pi 5 (8GB Cortex-A76) ¦ ¦ Arduino Uno R3 (ATmega328P) ¦ ¦-----------------------------------¦ ¦----------------------------------¦ ¦ • CSI Camera (640x480 @ 30 FPS) ¦ USB-Serial ¦ • Hardware Timer0 PWM (D5, D6) ¦ ¦ • YOLOv10n NCNN (NMS-Free) ¦ 115200 8N1 ¦ • 3-Tier Priority State Machine ¦ ¦ • Custom RDD2022 Pothole YOLO/CV ¦<----------->¦ • 2x HC-SR04P Ultrasonic Polling ¦ ¦ • KCF + HSV Torso Re-ID Tracker ¦ ¦ • L298N Dual H-Bridge Driver ¦ ¦ • Discrete PID Steering Engine ¦ ¦ • Sub-10ms Local Reflex Avoidance¦ +-----------------------------------+ +----------------------------------+ ^ ^ ¦ 5.1V / 5.0A USB-C ¦ 5.0V / 5.0A Reg. +-----------------------------------+ +----------------------------------+ ¦ boAt EnergyShroom PB400 Powerbank ¦ ¦ 2S 18650 Li-Ion (7.4V) + Buck ¦ +-----------------------------------+ +----------------------------------+

3.1 Complete Hardware Pin Allocation Matrix (Post-PWM Migration)

All hardware pins are governed by a single source of truth in arduino/lib/pins.h. The 4-wire PWM migration was completed and verified on 2026-09-30, moving ultrasonics off PWM pins to enable Timer0 motor control:

| Pin | Direction | Hardware Assignment | Functional Role & Timing Characteristics |
| --- | --- | --- | --- |
| D2 | INPUT | IR Sensor 1 | Front Left ground proximity digital input (HIGH/LOW) |
| D3 | INPUT | IR Sensor 2 | Front Right ground proximity input (Interrupt-capable INT1) |
| D4 | INPUT | US1 Echo (Front-Left) | Echo pulse width timing (HC-SR04P 3.3V/5V compatible) |
| D5 | OUTPUT | ENA (Left Motor PWM) | Timer0 (OC0B) 8-bit hardware PWM speed control (0–255, 980 Hz) |
| D6 | OUTPUT | ENB (Right Motor PWM) | Timer0 (OC0A) 8-bit hardware PWM speed control (0–255, 980 Hz) |
| D7 | OUTPUT | US2 Trigger (Front-Right) | 10 µs ultrasonic trigger pulse output |
| D8–D9 | OUTPUT | Motor IN1, IN2 | Left Motor direction bits (IN1=H/IN2=L: FWD; IN1=L/IN2=H: REV) |
| D10–D11 | OUTPUT | Motor IN3, IN4 | Right Motor direction bits (IN3=H/IN4=L: FWD; IN3=L/IN4=H: REV) |
| D12 | OUTPUT | US1 Trigger (Front-Left) | Moved from D5 during PWM migration to free Timer0 PWM pin |
| D13 | INPUT | US2 Echo (Front-Right) | Moved from D6 during PWM migration; on-board LED flickers (cosmetic) |

3.2 The 3-Tier Priority State Machine & Firmware Execution Model

To guarantee hard real-time safety, motor actuation is governed by an autonomous 3-tier finite state machine running at a deterministic 100 Hz loop rate (10 ms cycle time) on the ATmega328P:

Tier 1 (Highest Priority — Emergency Latches & Watchdog): PI_STOP command or serial communication timeout (>5000 ms watchdog). Forces ENA=0, ENB=0 (hard brake) and transitions to STATE_PI_STOPPED. All lower tier actions (including active Tier 2 avoidance maneuvers) are killed immediately. Movement remains blocked until an explicit PI_GO command is verified.

Tier 2 (Autonomous Ultrasonic Reflex Avoidance): Triggered locally if either front ultrasonic sensor reads < 20.0 cm during forward motion or turning. Executes a deterministic three-phase reflex: Phase 1 Reverse (400 ms at PWM 150) -> Phase 2 Skid-Turn away from the obstacle (500 ms at PWM 150) -> Phase 3 Forward Resume (2000 ms at saved cruising speed). An AVOID_COOLDOWN_MS of 500 ms suppresses hunting oscillations.

Tier 3 (Lowest Priority — Supervised Navigation Primitives): Executes high-level Pi commands (DRIVE <spd> <ms>, TURN <deg> <spd>, REV <spd> <ms>, STOP). Maintains zero command queuing—new Tier 3 commands instantly replace active commands.

4. Algorithms & Perception Pipeline Formulation [Rubric C: Algorithms Stated]

――――――――――――――――――――――――――――――――――――――――――――――――――――

4.1 Dual-Modality Negative Obstacle & Pothole Engine

Because horizontal acoustic sensors cannot detect road depressions, we developed a dual-modality computer vision pipeline:

Deep Learning Path (Custom RDD2022 YOLOv8n): Fine-tuned on the Road Damage Dataset 2022 (India D40 Pothole class). Operates with perspective corridor cropping to classify potholes in the driving path and outputs directional swerve directives (SWERVE_LEFT, SWERVE_RIGHT, EMERGENCY_STOP).

Classical Computer Vision Fallback (cv_detect_potholes): Executes in <4 ms on Pi 5 CPU without GPU dependency: crops bottom 55% of frame (ROI), applies 5x5 Gaussian smoothing, computes second-order spatial gradients via cv2.Laplacian (CV_64F), applies 13x13 elliptical morphological structuring (cv2.MORPH_ELLIPSE), and filters contours by area (900 < Area < 0.25 * ROI_Area) and aspect ratio (0.25 < w/h < 5.0).

4.2 Mathematical Formulations

• 1. Zone Danger Scoring Equation (avoidance.py):

Danger = Confidence × W × [ 0.4 + min(Area_box / Area_frame, 0.5) ]
where W = 1.0 for positive obstacles and W = 1.2 to 2.0 (POTHOLE_WEIGHT) for negative obstacles, reflecting the severe risk of chassis drop-off.

• 2. HSV Torso Appearance Model & Bhattacharyya Re-ID (tracker.py):

When the target person is occluded, the KCF tracker triggers a visual re-acquisition sweep using Bhattacharyya distance across normalized 2D HSV histograms:
d(H1, H2) = sqrt( 1 - (1 / sqrt(sum(H1)*sum(H2))) * sum_i(sqrt(H1(i) * H2(i))) )
Candidate boxes with correlation exceeding 0.72 are re-locked as the target owner.

• 3. Discrete PID Steering & Distance Visual Servoing (pid.py):

u(t) = Kp * e(t) + Ki * sum(e(t)*dt) + Kd * (e(t) - e(t-1)) / dt
where lateral error e_steer = (x_center_box - x_center_frame) / w_frame, and distance error e_dist = (h_box / h_frame - 0.30).

5. Economic Feasibility & Bill of Materials (BOM) [Rubric E: Technical Requirements & Real Costs]

――――――――――――――――――――――――――――――――――――――――――――――――――――

A critical strength of our engineering design is realistic economic analysis. In the Indian electronics retail market, the Raspberry Pi 5 (8GB) carries an official import price, customs duty, and GST that sets its local retail price between ₹15,500 and ₹16,000 INR (approx. $185 USD). However, because our system was built as an academic capstone, the high-value computing assets were sourced directly from the college robotics laboratory, drastically reducing student out-of-pocket expenditure.

| Subsystem | Component Specification | India Market Retail Price | Student Project Cost | Sourcing Status |
| --- | --- | --- | --- | --- |
| High-Level Brain (SBC) | Raspberry Pi 5 (8GB RAM, Quad Cortex-A76 @ 2.4GHz) | ₹15,800 ($188) | ₹0 (Offset) | College Lab Asset |
| Real-Time Controller | Arduino Uno R3 (ATmega328P DIP-28, 16MHz) | ₹1,100 ($13) | ₹0 (Offset) | College Lab Asset |
| Vision Sensor | Raspberry Pi Camera Module v2 (CSI 8MP / Global Shutter) | ₹2,400 ($29) | ₹0 (Offset) | College Lab Asset |
| Motor Driver | L298N Dual H-Bridge Heavy Duty Driver Module | ₹280 ($3.40) | ₹280 ($3.40) | Local Electronics |
| Chassis & Drive Train | 4WD Smart Robot Chassis + 4x TT DC Gear Motors (1:48) | ₹1,150 ($13.80) | ₹1,150 ($13.80) | Local Robotics Store |
| Distance Ranging | 2x HC-SR04P Ultrasonic (3.3V/5V) + 2x IR Proximity | ₹450 ($5.40) | ₹450 ($5.40) | Local Electronics |
| Motor Power Rail | 2S 18650 Li-Ion Pack (7.4V, 2500mAh) + LM2596 5V/5A Buck | ₹1,200 ($14.40) | ₹1,200 ($14.40) | Purchased New |
| Logic Power Rail | boAt EnergyShroom PB400 USB-C Power Bank (5.1V/5A Isolated) | ₹1,499 ($18.00) | ₹1,499 ($18.00) | Commercial Retail |
| Wiring & Power Bus | 16-pin Berg Strip header, F-to-M, M-to-M jumper wires | ₹350 ($4.20) | ₹350 ($4.20) | Lab Consumables |
| TOTAL SYSTEM VALUE | Full Autonomous 2-Layer Vehicle Assembly | ₹24,229 INR ($290) | ₹4,929 INR ($59) | Extremely Cost Effective |

• Key Financial Insight:

While commercial platforms like JetBot or JetRacer require an upfront retail commitment of ₹26,000–₹35,000 INR, our modular two-layer architecture allows educational institutions to leverage existing SBCs and microcontrollers. The net out-of-pocket expense to construct this research vehicle was strictly ₹4,929 INR (~$59 USD), demonstrating unmatched economic scalability for university robotics research.

6. Implementation Progress & Empirical Results [Rubric D: >50% Completed Work]

――――――――――――――――――――――――――――――――――――――――――――――――――――

In compliance with Rubric D (>50% completed implementation), more than 65% of the total project engineering work has been completed, physically deployed, and benchmarked across both layers:

| Subsystem | Implemented Modules & Artifacts | % Completed | Verification Status |
| --- | --- | --- | --- |
| Firmware & Motor Control | point_to_point_control.ino, dual_motor_speed_test.ino, pins.h (PWM migration) | 100% | Physically Verified on Bench (D5/D6 PWM) |
| Serial Protocol Bridge | PROTOCOL.md (115200 baud), motor_controller.py, sensors.py | 100% | Bench Verified (1.2 ms round trip latency) |
| Pothole Perception (ML) | best.pt, rdd2022_yolov8n.pt, run_detection.py (50 epochs on RDD2022) | 100% | Trained & Validated on video benchmark |
| Obstacle Perception & re-ID | detector.py (YOLOv10n NCNN), avoidance.py (zone danger), tracker.py (HSV re-ID) | 85% | Benchmarked at 22–26 ms on Pi 5 CPU |
| Autonomous Navigation & PID | main.py, pid.py, navigation.py, scenario specifications 01–10 | 75% | Integration Testing Underway (Tethered) |

6.1 Empirical Pothole Model Training Results (RDD2022 India)

A dedicated custom pothole detection model was trained for 50 epochs on the RDD2022 India dataset (pothole_detection/runs_rdd/custom_pothole_model/results.csv). The empirical metrics demonstrate stable convergence:

Training Convergence: Bounding box loss decreased from 2.018 (Epoch 1) to 1.711 (Epoch 50). Classification loss dropped from 6.068 to 1.915.

Detection Metrics: Final validation Precision achieved 54.0% with mAP@0.5 of 25.2% on unconstrained real-world Indian road asphalt cavities, sufficient for real-time directional swerve triggers.

Execution Latency: On-device YOLOv10n inference sustains 22–26 ms per frame (38–45 FPS) on the Raspberry Pi 5 CPU under NCNN, maintaining an end-to-end loop latency well within the 33 ms requirement.

7. Technical Requirements & Future Project Extensions [Rubric E: Future Extensions]

――――――――――――――――――――――――――――――――――――――――――――――――――――

7.1 Technical Operating Requirements

Host OS: Raspberry Pi OS Bookworm (64-bit Debian 12), Linux Kernel 6.6 AArch64.

Software Libraries: Python 3.11+, OpenCV 4.8+ (headless), Tencent NCNN (ARM NEON FP16), Ultralytics YOLOv10/v8, PySerial 3.5.

Microcontroller Toolchain: Arduino IDE / avr-gcc 7.3+, Baud Rate 115200 bps, ATmega328P 16 MHz 5V.

Thermal & Power Budget: Pi 5 actively cooled via Official Heatsink/Fan (steady-state 52°C under full NCNN load); isolated 5.1V/5A logic supply.

7.2 Prioritized Future Extensions Roadmap

1. Traffic Sign Recognition Integration (Option B): Integrate a Two-Stage Hierarchical Pipeline combining an upper-ROI color/geometry gate (<2 ms) with an NCNN MobileNetV4-Small classifier (<3.5 ms) to recognize 6 IRC track signs (go, right, park, red, green, crosswalk). This decoupled design ensures zero regression to obstacle avoidance.

2. IMU-Aided Yaw Dead-Reckoning (MPU-6050): Integrate an MPU-6050 6-DOF IMU on the Arduino I2C bus to measure true angular turning yaw, eliminating skid-steer wheel-slip drift during avoidance maneuvers.

3. Arduino Nano Hardware Form-Factor Migration: Migrate from the Uno R3 to an Arduino Nano (identical ATmega328P core, pinout, and timer registers), reducing microcontroller chassis footprint by 70%.

Formal Academic References [Rubric A: Citable Sources]

――――――――――――――――――――――――――――――――――――――――――――――――――――

[1] M. Müller and V. Koltun, "OpenBot: Turning Smartphones into Robots," in IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), 2020, pp. 6411–6418. arXiv:2008.10631.

[2] A. Wang, H. Chen, L. Liu, K. Chen, Z. Lin, J. Han, and G. Ding, "YOLOv10: Real-Time End-to-End Object Detection," arXiv preprint arXiv:2405.14458, 2024.

[3] G. V. et al., "IDD: A Dataset for Exploring Problems of Autonomous Navigation in Unconstrained Environments," in IEEE Winter Conference on Applications of Computer Vision (WACV), 2019, pp. 1743–1751.

[4] D. Arya et al., "Crowdsensing-based Road Damage Detection, Classification and Evaluation (RDD2022)," in IEEE International Conference on Big Data, 2022, pp. 6271–6278.

[5] Tencent, "NCNN: High-Performance Neural Network Inference Framework Optimized for Mobile Platforms," GitHub Repository, 2024. [Online]. Available: https://github.com/Tencent/ncnn

[6] J. F. Henriques, R. Caseiro, P. Martins, and J. Batista, "High-Speed Tracking with Kernelized Correlation Filters," IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 37, no. 3, pp. 583–596, Mar. 2015.

[7] W. Rosenthal, "Donkey Car: An Open Source DIY Self Driving Platform for Small Scale Cars," Donkey Car Documentation, 2023. [Online]. Available: https://docs.donkeycar.com/

[8] NVIDIA Corporation, "NVIDIA JetBot: An Open-Source AI Robot Platform Powered by NVIDIA Jetson Nano," NVIDIA Developer Zone, 2021. [Online]. Available: https://developer.nvidia.com/embedded/jetbot

[9] Indian Road Congress, "Code of Practice for Road Signs (IRC:67-2012)," New Delhi: Indian Roads Congress, 2012.

[10] P. Surve, A. Rana, and K. Shah, "Two-Layer Heterogeneous System Architecture and Serial Telemetry Protocol for Autonomous RC Vehicles," Capstone Documentation, NMIMS MPSTME, 2026.